Course			: Diploma in Electronics & Computer Engineering (EGDF20)

Module		: Connected System Design Project (EGE205)

Laboratory No.		: Lab 2a

Laboratory Title	: Display: Controlling the LED Segment using BeagleBone Black Wireless (BBBW)

Board

Objective	: To connect hardware click boards, install python library and write python code

to control different types of LED Segment click boards.

Hardware Boards	: BBBW Board with USB Cable		x1

: MikroBus Cape				x1

: BarGraph 2 Click			x1

: 7Seg Click				x1

Contents

- Use of Cape on BeagleBone Black Wireless (BBBW) Board

  - BBBW Board Cape Expansion Headers and Pinout

  - Connecting MikroBus Cape to the BBBW Board

- Use of Python Library on BeagleBone Black Wireless (BBBW) Board

  - Updating and Upgrading the Software Dependency Repositories in Debian OS

  - Installing the Blinka Python Library

  - Using the Blinka Python Library to Blink a LED

- Controlling the LED Segment using BeagleBone Black Wireless (BBBW) Board

  - Configuring the (BBBW) Board Boot-up Settings

  - Understanding of BarGraph 2 Click Hardware Connection

  - Controlling the BarGraph 2 Click using Python Code

  - Understanding of 7Seg Click Hardware Connection

  - Controlling the 7Seg Click using Python Code

  - Controlling both the BarGraph 2 and 7Seg Clicks using Python Code

# Use of Cape on BeagleBone Black Wireless (BBBW) Board

## BBBW Board Cape Expansion Headers and Pinout

BeagleBone Black Wireless (BBBW) board has two expansion headers P8 and P9 with each header provides 46 pins as shown in the Figure below. In total there are 92 pins available for use. Below shows the distribution of pins based on different pin functions:

Power / Ground / Reset	(VDD, GND)			: 18 pins

General Purpose Input / Output (GPIO)			: 19 pins

Analog Input (AIN)					: 7 pins

Pulse Width Modulation (PWM)				: 5 pins

Serial Peripheral Interface (SPI)				: 7 pins

Inter-Integrate Circuit (I2C)				: 2 pins

Universal Asynchronous Receiver Transmitter (UART)	: 4 pins

Embedded Multi-Media Controller (eMMC)		: 10 pins

Liquid-Crystal Display Interface (LCD)			: 20 pins

Figure 1.1a: BBBW Board Cape Expansion Headers and Pinout

## Connecting MikroBus Cape to the BBBW Board

The mikroBUS cape is an extension for BBBW board and introduces new forms of interaction with the outside world. This simple cape has four mikroBUS sockets that allow more than 130 different types of click boards (aka daughter boards) to be connected to the BBBW board. Additional functionality such as GSM, GPS, ZigBee or thunder detection, proximity and color sensing, hall current detection, speech recognition and many other can be quickly added to the BBBW board. To fully utilize the BBBW board pinout, two additional sets of jumpers allow you to:

- Switch the AN pin to GPIO on each mikroBUS socket.

(The cape has been configured to use AN pin as analog pin)

- Choose whether to enable UART pins on the first socket or SPI pins on the third socket.

(The cape has been configured to use it as SPI pins on the third socket)

The mikroBUS cape and its respective schematic are shown in the Figure below.

Figure 1.2a: MikroBUS Cape and Schematic

- Connect the mikroCape to the BBBW board’s two expansion headers P8 and P9 with the correct orientation as show in the Figure below.

Figure 1.2b: Connecting MikroBUS Cape onto the BBBW Board

# Use of Python Library on BeagleBone Black Wireless (BBBW) Board

## Updating and Upgrading the Software Dependency Repositories in Debian OS

Advanced Package Tool (APT), is a free-software user interface that works with core libraries to handle the installation and removal of software on Debian, Ubuntu, and related Linux distributions. APT also includes command-line programs for dealing with packages, which use the library. Three such programs are apt, apt-get and apt-cache.

- Log in to the BBBW board through SSH using the default username “debian” and password “temppwd” as shown in the Figure below.

Figure 2.1a: Logging in to BBBW Board

- Type in the command “iwconfig” and hit the “Enter” key.  Ensure that the wlan0 is connected to a particular Wi-Fi’s SSID. In this case, the BBBW board has been connected to SSID: “S435-CSDL_dev” as shown in the Figure below.

Figure 2.1b: The “iwconfig” Command

- Type in the command “date” and hit the “Enter” key. This command returns the default UTC date and time configured in the system as shown in the Figure below.

Figure 2.1c: The “date” Command

- Type in the command “sudo date -s “25 Oct 2021 17:10:00”” (Modify the command to use the current UTC date and time) and hit the “Enter” key. This command updates the system date and time according to the current UTC date and time as shown in the Figure below.

Figure 2.1d: The “date -s” Command

- Type in the command “sudo apt-get update” and hit the “Enter” key. Type in the password “temppwd” and hit the “Enter” key again. This command synchronizes the Linux package index files from the sources and may take up to 5 minutes to complete the synchronization process as shown in the Figure below.

Figure 2.1e: The “apt-get update” Command

- Type in the command “sudo apt-get update” and hit the “Enter” key again. The completion of the synchronization process is shown in the Figure below.

Figure 2.1f: The “apt-get update” Command

- The BBBW board has just been successfully updated!

[DO NOT DO THE STEPS BELOW] Additional Information: Upgrade the BBBW board - Type in the command “sudo apt-get upgrade” and hit the “Enter” key. This command installs the latest versions of the packages currently installed on the BBBW board. The installed packages which have new packages available will be retrieved and installed as shown in the Figure below. (Note: An update needs to be performed before the upgrade, so that apt-get knows that new versions of packages are available for installation) Figure 2.1g: The “apt-get upgrade” Command - Type “y” and hit the “Enter” key to continue the upgrading process and it may take up to 15 minutes to complete the upgrading process as shown in the Figure below. Figure 2.1h: Continue the Upgrading Process - Type in the command “sudo apt-get upgrade” and hit the “Enter” key again. Type in the password “temppwd” and hit the “Enter” key again if prompted. The completion of the upgrading process is shown in the Figure below. Figure 2.1i: The “apt-get upgrade” Command

## Installing the Blinka Python Library

Blinka is a python library that brings CircuitPython APIs to single board computers (SBCs) such as BBBW board. It provides a layer that translates the CircuitPython hardware API to whatever library the Linux board provides as shown in the Figure below.

There are many approaches to install python library. The approach of “installing from source” will be used throughout this module. This approach requires a copy of the python library source code to be cloned into the BBBW board before installing it. Hence, a folder named “PythonLibrary” will be created in the BBBW board to keep all the python library source code.

Figure 2.2a: Blinka Python Library

- Type in the command “pwd” and hit the “Enter” key. It is observed that the current working directory “/home/debian” is returned as shown in the Figure below.

Figure 2.2b: Current Working Directory

- Type in the command “mkdir PythonLibrary” and hit the “Enter” key. This command creates a folder named PythonLibrary inside the current working directory as shown in the Figure below.

Figure 2.2c: Creating PythonLibrary Folder

- Type in the command “ls” and hit the “Enter” key. It is observed that the PythonLibrary folder has been created as shown in the Figure below.

Figure 2.2d: PythonLibrary Folder is Created

- Type in the command “cd PythonLibrary” and hit the “Enter” key. This command accesses the PythonLibrary folder as shown in the Figure below.

Figure 2.2e: Accessing the PythonLibrary Folder

- Type in the command “git clone https://github.com/nypege205/Adafruit_Blinka.git” and hit the “Enter” key. Type in the github username “nypege205” and password “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” and hit the “Enter” key again. This command clones a copy of the Adafruit_Blinka Python Library repository from github as shown in the Figure below. (Note: Type the git clone command again if the follow error appear -> “fatal: unable to access 'https://github.com/nypege205/Adafruit_Blinka.git/': server certificate verification failed. CAfile: none CRLfile: none”)

Figure 2.2f: Cloning the Blinka Python Library

- Type in the command “ls” and hit the “Enter” key. The Adafruit_Blinka Python Library folder is returned as shown in the Figure below.

Figure 2.2g: Listing the Adafruit_Blinka Python Library Folder

- Type in the command “cd Adafruit_Blinka” and hit the “Enter” key. This is to access the Adafruit_Blinka folder as shown in the Figure below.

Figure 2.2h: Accessing the Adafruit_Blinka Folder

- Type in the command “sudo python3 setup.py install” and hit the “Enter” key. This command installs the Adafruit_Blinka Python Library. The installation may take up to 5 minutes. The completion of the installation process is shown in the Figure below.

Figure 2.2i: Completion of Adafruit_Blinka Python Library Installation

- Type in the command “sudo python3 -m pip install --upgrade --force-reinstall adafruit-blinka Adafruit-PlatformDetect” and hit the “Enter” key.

- The BBBW board has just been successfully installed with the Adafruit_Blinka Python Library.

## Using the Blinka Python Library to Blink a LED

- Ensure that the BBBW board is powered up and connected to the computer through a USB cable. Open the web browser (preferably Chrome browser) and type “http://192.168.7.2:3000” in the address bar as shown in the Figure.

Figure 2.3a: Cloud9 IDE

- Right click on the folder “MyFirstPythonProject” and select the “New File” from the drop-down menu to create a new python file. Name the file as “Blinky.py” as shown in the Figure below.

Figure 2.3b: Creating a New File

- Double click on the newly created file “Blinky.py” and enter the following code into the file under the Editor section as shown in the Figure below.

import time import Adafruit_BBIO.GPIO as GPIO GPIO.setup("USR0", GPIO.OUT) while True: GPIO.output("USR0", GPIO.HIGH) time.sleep(0.5) GPIO.output("USR0", GPIO.LOW) time.sleep(0.5)

Figure 2.3c: Blinky Python Code

- Click on the “Run” button located beside the Menu Tab to execute the “Blinky.py” file. It is observed that the blue LED (USR0) located beside the reset button is blinking at the rate of 0.5s as shown in the Figure below.

Figure 2.3d: Location of the Blue LED (USR0)

# Controlling the LED Segment using BeagleBone Black Wireless (BBBW) Board

## Configuring the (BBBW) Board Boot-up Settings

The BBBW board includes 2 SPIs namely SPI0 and SPI1. Both SPI0 and SPI1 cannot be used at runtime and SPI1 is not available by default as the HDMI interface is utilizing one of the pins. Hence, the following actions need to be done to enable both SPI0 and SPI1 for use.

- Enable both SPI0 and SPI1 at boot.

- Disable HDMI interface at boot.

- Log in to the BBBW board through SSH using the default username “debian” and password “temppwd”.

- Type in the command “sudo nano /boot/uEnv.txt“ and hit the “Enter” key. Type in the password “temppwd” and hit the “Enter” key again. This command accesses and opens the file uEnv.txt  found in the /boot folder for editing using “nano” which is a simple command-line text editor included in most Linux installations as shown in the Figure below.

Figure 3.1a: Assessing the Boot File uEnv.txt

- Add the following 2 lines of text into the file uEnv.txt. This is to enable both SPI0 and SPI1 at boot as shown in the Figure below.

uboot_overlay_addr4=/lib/firmware/BB-SPIDEV0-00A0.dtbo uboot_overlay_addr5=/lib/firmware/BB-SPIDEV1-00A0.dtbo

Figure 3.1b: Enabling Both SPI0 and SPI1 at Boot

- Remove the “#” for the following 2 lines of text in the file uEnv.txt. This is to disable HDMI interface at boot as shown in the Figure below.

Figure 3.1c: Disabling the HDMI Interface At Boot

- Press the “Ctrl + X” key followed by hitting the “y” and “Enter” key to save the modified file and exit the nano text-editor as shown in the Figure below

Figure 3.1d: Exiting the nano Text-Editor

- Type in the command “sudo reboot” and hit the “Enter” key. This is to reboot the BBBW board for the new settings to take effect on the next boot-up. Please note that the SSH connection will be dropped.

## Understanding of BarGraph 2 Click Hardware Connection

BarGraph 2 Click is a 10-segment bar graph display click, which uses a high-quality, multicolor bar graph LED display. The bar graph display is a very popular device for displaying various properties, whether it be an audio level, current/voltage level, position of the encoder, and generally any property that can be displayed in a form of a bar graph. The segments of the on-board bar graph LED display are bright and uniformly colored, providing a nice and clean visual feedback.

Each segment is composed of green and red colored LEDs, making it possible to have various important states marked in different color. It can use green, red, and a combination of these two - resulting in having amber colored segments. For example, a VU-meter can have its highest values shown in red (peaks), middle in yellow and the lowest in green; the position of the encoder can be marked with a single red segment, on a green background, and so on. The bar graph display light intensity can be dimmed, by applying a PWM signal. The Click board offers a lot of possibilities for building a custom bar graph application - VU meters, status indicators, various types of gauges, and similar.

The BarGraph 2 Click, and its respective schematic are shown in the Figure below.

Figure 3.2a: BarGraph 2 Click and Schematic

- Connect the BarGraph 2 Click to the mikroBUS cape and BBBW board as shown in the Figure below.

Figure 3.2b: Connecting BarGraph 2 Click to mikroBUS Cape and BBBW Board

## Controlling the BarGraph 2 Click using Python Code

- Ensure that the BBBW board is powered up and connected to the computer through a USB cable. Open the web browser (preferably Chrome browser) and type “http://192.168.7.2:3000” in the address bar as shown in the Figure below.

Figure 3.3a: Cloud9 IDE

- Right click on the folder “MyFirstPythonProject” and select the “New File” from the drop-down menu to create a new python file. Name the file as “BarGraph2.py”.

- Double click on the newly created file “BarGraph2.py” and enter the following code into the file under the Editor section.

import time from Adafruit_BBIO.SPI import SPI import Adafruit_BBIO.GPIO as GPIO def BarGraph2Init(): GPIO.setup("P9_14", GPIO.OUT) GPIO.setup("P9_12", GPIO.OUT) GPIO.output("P9_14", GPIO.HIGH) GPIO.output("P9_12", GPIO.HIGH) L_Spi1 = SPI(1,0) L_Spi1.mode = 0 return L_Spi1 def BarGraph2Display(L_Spi1, L_NumberOfBar): if L_NumberOfBar == 0: L_Spi1.writebytes([0x00, 0x00, 0x00]) if L_NumberOfBar == 1: L_Spi1.writebytes([0x00, 0x00, 0x01]) if L_NumberOfBar == 2: L_Spi1.writebytes([0x00, 0x00, 0x03]) if L_NumberOfBar == 3: L_Spi1.writebytes([0x00, 0x00, 0x07]) if L_NumberOfBar == 4: L_Spi1.writebytes([0x00, 0x00, 0x0F]) if L_NumberOfBar == 5: L_Spi1.writebytes([0x00, 0x40, 0x1F]) if L_NumberOfBar == 6: L_Spi1.writebytes([0x00, 0xC0, 0x3F]) if L_NumberOfBar == 7: L_Spi1.writebytes([0x01, 0xC0, 0x7F]) if L_NumberOfBar == 8: L_Spi1.writebytes([0x03, 0xC0, 0xFF]) if L_NumberOfBar == 9: L_Spi1.writebytes([0x07, 0xC0, 0xFF]) if L_NumberOfBar == 10: L_Spi1.writebytes([0x0F, 0xC0, 0xFF]) G_NumberOfBar = 0 G_Spi1 = BarGraph2Init() while True: BarGraph2Display(G_Spi1, G_NumberOfBar) G_NumberOfBar += 1 if G_NumberOfBar == 11: G_NumberOfBar = 0 time.sleep(0.3)

- The code above uses the functions explained below to make the display on the BarGraph 2 Click possible.

| Function | Description |
| --- | --- |
| BarGraph2Init() | To initialize the required GPIO and communication channel (SPI) and return the initialized SPI instance to the function call. |
| BarGraph2Display(L_Spi1, L_NumberOfBar) | To control the display on the BarGraph 2 Click. The function needs to be supplied with an initialized SPI instance to L_Spi1 and an integer number ranging from 0-10 to L_NumberOfBar. |

- The BarGraph 2 Click consists of 10 yellow and 10 red leds. The yellow and red led can be turned on at the same time to produce an amber color as shown in the Figure below.

Figure 3.3b: Cloud9 IDE

- Click on the “Run” button located beside the Menu Tab to execute the “BarGraph2.py” file. It is observed that the LED bars located on the BarGraph2 Click is turned on progressively from left to right with the color changing from green to amber then red and repeats itself.

## Understanding of 7Seg Click Hardware Connection

7seg Click is an accessory board in mikroBUS form factor. It’s a compact and easy solution for adding 2-digit seven-segment display to your device.

7Seg Click and its respective schematic are shown in the Figure below.

Figure 3.4a: 7Seg Click and Schematic

- Connect the 7Seg Click to the mikroBUS cape and BBBW board as shown in the Figure below.

Figure 3.4b: Connecting 7Seg Click to mikroBUS Cape and BBBW Board

## Controlling the 7Seg Click using Python Code

- Ensure that the BBBW board is powered up and connected to the computer through a USB cable. Open the web browser (preferably Chrome browser) and type “http://192.168.7.2:3000” in the address bar.

- Right click on the folder “MyFirstPythonProject” and select the “New File” from the drop-down menu to create a new python file. Name the file as “7Seg.py”.

- Double click on the newly created file “7Seg.py” and enter the following code into the file under the Editor section.

import time from Adafruit_BBIO.SPI import SPI import Adafruit_BBIO.GPIO as GPIO def SevenSegInit(): GPIO.setup("P8_19", GPIO.OUT) GPIO.setup("P8_14", GPIO.OUT) GPIO.output("P8_19", GPIO.HIGH) GPIO.output("P8_14", GPIO.HIGH) L_Spi0 = SPI(0,0) L_Spi0.mode = 0 return L_Spi0 def SevenSegDisplay(L_Spi0, L_Number): DigitList = [0x7E, 0x0A, 0xB6, 0x9E, 0xCA, 0xDC, 0xFC, 0x0E, 0xFE, 0xDE] OnesDigit = L_Number % 10 TensDigit = L_Number / 10 L_Spi0.writebytes([DigitList[int(OnesDigit)], DigitList[int(TensDigit)]]) G_Number = 0 G_Spi0 = SevenSegInit() while True: SevenSegDisplay(G_Spi0, G_Number) G_Number += 1 if G_Number == 100: G_Number = 0 time.sleep(0.3)

- The code above uses the functions explained below to make the display on the 7Seg Click possible.

| Function | Description |
| --- | --- |
| SevenSegInit() | To initialize the required GPIO and communication channel (SPI) and return the initialized SPI instance to the function call. |
| SevenSegDisplay(L_Spi0, L_Number) | To control the display on the 7Seg Click. The function needs to be supplied with an initialized SPI instance to G_Spi0 and an integer number ranging from 0-99 to L_Number. |

- Click on the “Run” button located beside the Menu Tab to execute the “7Seg.py” file. It is observed that the 7 segments digits located on the 7Seg Click are counting from 00-99 and repeats itself.

## Controlling both the BarGraph 2 and 7Seg Clicks using Python Code

- Connect both the BarGraph 2 and 7Seg Clicks to the mikroBUS cape and BBBW board as shown in the Figure below.

Figure 3.6a: Connecting both The BarGraph 2 and 7Seg Click to mikroBUS Cape and BBBW Board

- Ensure that the BBBW board is powered up and connected to the computer through a USB cable. Open the web browser (preferably Chrome browser) and type “http://192.168.7.2:3000” in the address bar.

- Right click on the folder “MyFirstPythonProject” and select the “New File” from the drop-down menu to create a new python file. Name the file as “BarGraph2_7Seg.py”.

- Double click on the newly created file “BarGraph2_7Seg.py” and start entering code into the file under the Editor section.

- Enter the code that imports all the necessary library to be used in the program as shown below.

import time from Adafruit_BBIO.SPI import SPI import Adafruit_BBIO.GPIO as GPIO

- Enter the code of the 4 functions to be called in the program as shown below.

def BarGraph2Init(): GPIO.setup("P9_14", GPIO.OUT) GPIO.setup("P9_12", GPIO.OUT) GPIO.output("P9_14", GPIO.HIGH) GPIO.output("P9_12", GPIO.HIGH) L_Spi1 = SPI(1,0) L_Spi1.mode = 0 return L_Spi1 def SevenSegInit(): GPIO.setup("P8_19", GPIO.OUT) GPIO.setup("P8_14", GPIO.OUT) GPIO.output("P8_19", GPIO.HIGH) GPIO.output("P8_14", GPIO.HIGH) L_Spi0 = SPI(0,0) L_Spi0.mode = 0 return L_Spi0 def BarGraph2Display(L_Spi1, L_NumberOfBar): if L_NumberOfBar == 0: L_Spi1.writebytes([0x00, 0x00, 0x00]) if L_NumberOfBar == 1: L_Spi1.writebytes([0x00, 0x00, 0x01]) if L_NumberOfBar == 2: L_Spi1.writebytes([0x00, 0x00, 0x03]) if L_NumberOfBar == 3: L_Spi1.writebytes([0x00, 0x00, 0x07]) if L_NumberOfBar == 4: L_Spi1.writebytes([0x00, 0x00, 0x0F]) if L_NumberOfBar == 5: L_Spi1.writebytes([0x00, 0x40, 0x1F]) if L_NumberOfBar == 6: L_Spi1.writebytes([0x00, 0xC0, 0x3F]) if L_NumberOfBar == 7: L_Spi1.writebytes([0x01, 0xC0, 0x7F]) if L_NumberOfBar == 8: L_Spi1.writebytes([0x03, 0xC0, 0xFF]) if L_NumberOfBar == 9: L_Spi1.writebytes([0x07, 0xC0, 0xFF]) if L_NumberOfBar == 10: L_Spi1.writebytes([0x0F, 0xC0, 0xFF]) def SevenSegDisplay(L_Spi0, L_Number): OnesDigit = L_Number % 10 TensDigit = L_Number / 10 L_Spi0.writebytes([DigitList[int(OnesDigit)], DigitList[int(TensDigit)]])

- Enter the main code that call the 4 functions to control both the BarGraph 2 and 7Seg Click as shown below.

G_NumberOfBar = 1 G_Number = 0 NumCounter = 0 DigitList = [0x7E, 0x0A, 0xB6, 0x9E, 0xCA, 0xDC, 0xFC, 0x0E, 0xFE, 0xDE] G_Spi0 = SevenSegInit() G_Spi1 = BarGraph2Init() while True: SevenSegDisplay(G_Spi0, G_Number) BarGraph2Display(G_Spi1, G_NumberOfBar) G_Number += 1 if G_Number == 100: G_Number = 0 NumCounter += 1 if NumCounter == 10: NumCounter = 0 G_NumberOfBar += 1 if G_NumberOfBar == 11: G_NumberOfBar = 1 time.sleep(0.3)

- Click on the “Run” button located beside the Menu Tab to execute the “BarGraph2_7Seg.py” file.

- Observe and compare the output on both the BarGraph 2 and 7Seg Clicks with your teammates and consult your lecturer for advice if it is not the same.

- Write the program output in the white box below for future reference if needed.

Right click and select “New comment” to insert your program as a comment.

Congratulations! You have successfully completed the Lab2a. Good job! Take a good break and stay tune for next lab. More exciting lab exercises coming to you!