Course			: Diploma in Electronics & Computer Engineering (EGDF20)

Module		: Connected System Design Project (EGE205)

Laboratory No.		: Lab 2b

Laboratory Title	: Display: Controlling the LED Matrix and OLED using BeagleBone Black Wireless

(BBBW) Board

Objective	: To connect hardware click boards, install python library and write python code

to control different types of display click boards.

Hardware Boards	: BBBW Board with USB Cable		x1

: MikroBus Cape				x1

: 8x8 R Click				x1

: OLED B Click				x1

Contents

- Controlling the LED Matrix using BeagleBone Black Wireless (BBBW) Board

  - Understanding of 8x8 (R/G/Y/B) Click Hardware Connection

  - Controlling the 8x8 (R/G/Y/B) Click using Python Code

- Controlling the OLED using BeagleBone Black Wireless (BBBW) Board

  - Understanding of OLED (W/B) Click Hardware Connection

  - Controlling the OLED (W/B) Click using Python Code

  - Controlling both the 8x8 R and OLED B Clicks using Python Code

# Controlling the LED Maxtrix using BeagleBone Black Wireless (BBBW) Board

## Understanding of 8x8 (R/G/Y/B) Click Hardware Connection

8x8 R Click is a 64 LED matrix display Click board, composed of SMD LEDs organized in 8 rows by 8 columns. It has a digital brightness control in 16 steps, it can control every LED in the display matrix independently, it blanks the display on power up to eliminate glitches and it requires a single resistor to control the current through all the LEDs at once, which simplifies the design. 8x8 R Click uses a fast SPI communication protocol, allowing fast display response and no lag.

8x8 R Click can be used as a display or signalization output for a range of applications that are designed to display various information or graphics on the matrix LED display.

The 8x8 R Click, and its respective schematic are shown in the Figure below.

Figure 1.1a: 8x8 R Click and Schematic

- Connect the 8x8 R Click to the mikroBUS cape and BBBW board as shown in the Figure below.

Figure 1.1b: Connecting 8x8 R Click to mikroBUS Cape and BBBW Board

## Controlling the 8x8 R Click using Python Code

- Ensure that the BBBW board is powered up and connected to the computer through a USB cable. Open the web browser (preferably Chrome browser) and type “http://192.168.7.2:3000” in the address bar.

- Right click on the folder “MyFirstPythonProject” and select the “New File” from the drop-down menu to create a new python file. Name the file as “8x8r.py”.

- Double click on the newly created file “8x8r.py” and enter the following code into the file under the Editor section.

from Adafruit_BBIO.SPI import SPI G_SmileyHappyFace = [0b00111100, 0b01000010, 0b10101001, 0b10000101, 0b10000101, 0b10101001, 0b01000010, 0b00111100] def LedMatrix8x8ClickInit(): L_Spi1 = SPI(1,0) L_Spi1.mode = 0 L_Spi1.writebytes([0x09, 0x00]) L_Spi1.writebytes([0x0A, 0x01]) L_Spi1.writebytes([0x0B, 0x07]) L_Spi1.writebytes([0x0C, 0x01]) return L_Spi1 def PrintDisplay(L_Spi1, DisplayList): L_Spi1.writebytes([0x01, DisplayList[0]]) L_Spi1.writebytes([0x02, DisplayList[1]]) L_Spi1.writebytes([0x03, DisplayList[2]]) L_Spi1.writebytes([0x04, DisplayList[3]]) L_Spi1.writebytes([0x05, DisplayList[4]]) L_Spi1.writebytes([0x06, DisplayList[5]]) L_Spi1.writebytes([0x07, DisplayList[6]]) L_Spi1.writebytes([0x08, DisplayList[7]]) G_Spi1 = LedMatrix8x8ClickInit() PrintDisplay(G_Spi1, G_SmileyHappyFace)

- The code above uses the functions explained below to make the display on the 8x8 R Click possible.

| Function | Description |
| --- | --- |
| LedMatrix8x8ClickInit() | To initialize the required communication channel (SPI) and return the initialized SPI instance to the function call. |
| PrintDisplay(L_Spi1, DisplayList) | To print the display on the 8x8 R Click. The function needs to be supplied with an initialized SPI instance to L_Spi1 and a list that contains the information to be printed. |

- Click on the “Run” button located beside the Menu Tab to execute the “8x8r.py” file. It is observed that a smiley Happy Face is printed on the 8x8 R Click.

- Try creating a new list loaded with different bits information to display different graphics on the 8x8 R Click and show it to your lecturer.

# Controlling the OLED (W/B) using BeagleBone Black Wireless (BBBW) Board

## Understanding of OLED (W/B) Click Hardware Connection

OLED B Click carries a 96 x 39px blue monochrome passive matrix OLED display. The display is bright, has a wide viewing angle and low power consumption. To drive the display, OLED B click features an SSD1306 controller. Its built-in functionalities include contrast control, normal or inverse image display, vertical and horizontal scrolling functions and more. OLED B click can communicate with the target board MCU either through SPI or I2C mikroBUS lines. You switch between output options by resoldering the onboard SEL COMM jumpers (J1, J2 and J3) to the appropriate position.

OLED B Click and its respective schematic are shown in the Figure below.

Figure 2.1a: OLED B Click and Schematic

- Connect the OLED B Click to the mikroBUS cape and BBBW board as shown in the Figure below.

Figure 2.1b: Connecting OLED B Click to mikroBUS Cape and BBBW Board

## Controlling the OLED (W/B) Click using Python Code

Downloading and Installing Adafruit_CircuitPython_SSD1306 Python Library

- Log in to the BBBW board through SSH using the default username “debian” and password “temppwd”.

- Type in the command “iwconfig” and hit the “Enter” key to ensure that the wlan0 is connected to a particular Wi-Fi’s SSID.

- Type in the command “pwd” and hit the “Enter” key. It is observed that the current working directory “/home/debian” is returned.

- Type in the command “ls” and hit the “Enter” key. It is observed that the PythonLibrary folder has been created earlier.

- Type in the command “cd PythonLibrary” and hit the “Enter” key to accesses the PythonLibrary folder.

- Type in the command “git clone https://github.com/nypege205/Adafruit_CircuitPython_SSD1306.git” and hit the “Enter” key. Type in the github username “nypege205” and password “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” and hit the “Enter” key again to clones a copy of the Adafruit_CircuitPython_SSD1306 Python Library repository from github.

- Type in the command “ls” and hit the “Enter” key. The Adafruit_CircuitPython_SSD1306 Python Library folder is returned.

- Type in the command “cd Adafruit_CircuitPython_SSD1306” and hit the “Enter” key to access the Adafruit_CircuitPython_SSD1306 folder.

- Type in the command “sudo python3 setup.py install” and hit the “Enter” key to install the Adafruit_CircuitPython_SSD1306 Python Library. Please note that the installation may take up to 5 minutes to complete the whole process.

Installing Pillow Python Library using Pip

- Type in the command “python3 -m pip install --upgrade Pillow” and hit the “Enter” key to install the Pillow Python Library using pip which is the package installer for Python.

Installing libopenjp2-7 (JPEG 2000 image compression/decompression library) using apt-get

- Type in the command “sudo apt-get install libopenjp2-7” and hit the “Enter” key to install the libopenjp2-7 Python Library using apt-get.

Executing the Python Program

- Ensure that the BBBW board is powered up and connected to the computer through a USB cable. Open the web browser (preferably Chrome browser) and type “http://192.168.7.2:3000” in the address bar.

- Right click on the folder “MyFirstPythonProject” and select the “New File” from the drop-down menu to create a new python file. Name the file as “Oledb.py”.

- Double click on the newly created file “Oledb.py” and enter the following code into the file under the Editor section.

import board import busio import digitalio import adafruit_ssd1306 from board import SCL, SDA from PIL import Image, ImageDraw, ImageFont def OLEDClickInit(): Pin_DC = digitalio.DigitalInOut(board.P9_16) Pin_DC.direction = digitalio.Direction.OUTPUT Pin_DC.value = False Pin_RESET = digitalio.DigitalInOut(board.P9_23) Pin_RESET.direction = digitalio.Direction.OUTPUT Pin_RESET.value = True L_I2c = busio.I2C(SCL, SDA) return L_I2c G_I2c = OLEDClickInit() Display = adafruit_ssd1306.SSD1306_I2C(64, 32, G_I2c, addr=0x3C) ImageObj = Image.new("1", (Display.width, Display.height)) Draw = ImageDraw.Draw(ImageObj) Draw.rectangle((0, 0, Display.width - 1, Display.height - 1), outline=1, fill=0) Font = ImageFont.load_default() Text = "I Love NYP" Draw.text((2, 10), Text, font=Font, fill=1) Display.image(ImageObj) Display.show()

- The code above uses the functions explained below to make the display on the OLED B Click possible.

| Function | Description |
| --- | --- |
| OLEDClickInit() | To initialize the required GPIO and communication channel (I2C) and return the initialized I2C instance to the function call. |

- Click on the “Run” button located beside the Menu Tab to execute the “Oledb.py” file. It is observed that a rectangle outline is printed with a line of text “I Love NYP” printed inside.

- Try modifying the code to change the text and draw a smaller rectangle and display it on the OLED B Click and show it to your lecturer.

## Controlling both the 8x8 R and OLED B Clicks using Python Code

- Connect both the 8x8 R and OLED B Clicks to the mikroBUS cape and BBBW board as shown in the Figure below.

Figure 2.3a: Connecting both The 8x8 R and OLED B Click to mikroBUS Cape and BBBW Board

- Ensure that the BBBW board is powered up and connected to the computer through a USB cable. Open the web browser (preferably Chrome browser) and type “http://192.168.7.2:3000” in the address bar.

- Right click on the folder “MyFirstPythonProject” and select the “New File” from the drop-down menu to create a new python file. Name the file as “8x8_Oled.py”.

- Double click on the newly created file “8x8_Oled.py” and start entering code into the file under the Editor section.

- Enter the code that imports all the necessary library to be used in the program as shown below.

import time import board import busio import digitalio import adafruit_ssd1306 from board import SCL, SDA from PIL import Image, ImageDraw, ImageFont from Adafruit_BBIO.SPI import SPI

- Enter the code of the 4 lists to be used in the program as shown below.

G_ClearDisplay = [0b00000000, 0b00000000, 0b00000000, 0b00000000, 0b00000000, 0b00000000, 0b00000000, 0b00000000] G_CharacterN = [0b00000000, 0b01111110, 0b00000100, 0b00001000, 0b00010000, 0b00100000, 0b01111110, 0b00000000] G_CharacterY = [0b00000000, 0b00000000, 0b01100000, 0b00010000, 0b00001110, 0b00010000, 0b01100000, 0b00000000] G_CharacterP = [0b00000000, 0b00000000, 0b00110000, 0b01001000, 0b01001000, 0b01111110, 0b00000000, 0b00000000]

- Enter the code of the 3 functions to be called in the program as shown below.

def OLEDClickInit(): Pin_DC = digitalio.DigitalInOut(board.P9_16) Pin_DC.direction = digitalio.Direction.OUTPUT Pin_DC.value = False Pin_RESET = digitalio.DigitalInOut(board.P9_23) Pin_RESET.direction = digitalio.Direction.OUTPUT Pin_RESET.value = True L_I2c = busio.I2C(SCL, SDA) return L_I2c def LedMatrix8x8ClickInit(): L_Spi1 = SPI(1,0) L_Spi1.mode = 0 L_Spi1.writebytes([0x09, 0x00]) L_Spi1.writebytes([0x0A, 0x01]) L_Spi1.writebytes([0x0B, 0x07]) L_Spi1.writebytes([0x0C, 0x01]) return L_Spi1 def PrintDisplay(L_Spi1, DisplayList): L_Spi1.writebytes([0x01, DisplayList[0]]) L_Spi1.writebytes([0x02, DisplayList[1]]) L_Spi1.writebytes([0x03, DisplayList[2]]) L_Spi1.writebytes([0x04, DisplayList[3]]) L_Spi1.writebytes([0x05, DisplayList[4]]) L_Spi1.writebytes([0x06, DisplayList[5]]) L_Spi1.writebytes([0x07, DisplayList[6]]) L_Spi1.writebytes([0x08, DisplayList[7]])

- Enter the main code that call the 4 functions to control both the 8x8 R and OLED B Click as shown below.

G_I2c = OLEDClickInit() G_Spi1 = LedMatrix8x8ClickInit() Display = adafruit_ssd1306.SSD1306_I2C(64, 32, G_I2c, addr=0x3C) ImageObj = Image.new("1", (Display.width, Display.height)) Draw = ImageDraw.Draw(ImageObj) Draw.rectangle((0, 0, Display.width - 1, Display.height - 1), outline=1, fill=0) Font = ImageFont.load_default() while True: Draw.text((10, 10), 'N', font=Font, fill=1) Display.image(ImageObj) Display.show() PrintDisplay(G_Spi1, G_CharacterN) time.sleep(1) Draw.text((20, 10), 'Y', font=Font, fill=1) Display.image(ImageObj) Display.show() PrintDisplay(G_Spi1, G_CharacterY) time.sleep(1) Draw.text((30, 10), 'P', font=Font, fill=1) Display.image(ImageObj) Display.show() PrintDisplay(G_Spi1, G_CharacterP) time.sleep(1) Draw.rectangle((5, 10, 40, 20), outline=0, fill=0) Display.image(ImageObj) Display.show() PrintDisplay(G_Spi1, G_ClearDisplay) time.sleep(1)

- Click on the “Run” button located beside the Menu Tab to execute the “8x8_Oled.py” file.

- Observe and compare the output on both the 8x8 R and OLED Clicks with your teammates and consult your lecturer for advice if it is not the same.

- Write the program output in the white box below for future reference if needed.

Right click and select “New comment” to insert your program as a comment.

Congratulations! You have successfully completed the Lab2b. Good job! Take a good break and stay tune for next lab. More exciting lab exercises coming to you!