Course			: Diploma in Electronics and Computer Engineering (EGDF20)

Module		: Connected System Design Project (EGE205)

Laboratory No.		: Lab 4a

Laboratory Title	: Sensor and Actuator: Reading the Digital Sensor Data and Controlling the

Actuator using BeagleBone

Objective	: To connect hardware click boards and write python code to read data from

different types of sensors and control different types of actuators.

Hardware Boards	: BBBW Board			x1

: MikroBus Cape			x1

: Motion Click			x1

: Vibra Sense Click		x1

: IR Eclipse Click			x1

: Reed Click			x1

: Buzz 2 Click			x1

Contents

- Reading the Digital Sensors Data using BeagleBone Black Wireless (BBBW) Board

  - Understanding of Motion Click Hardware Connection

  - Reading Digital Data from Motion Click using Python Code

  - Understanding of Vibra Sense Click Hardware Connection

  - Reading Digital Data from Vibra Sense Click using Python Code

  - Understanding of IR Eclipse Click Hardware Connection

  - Reading Digital Data from IR Eclipse Click using Python Code

  - Understanding of Reed Click Hardware Connection

  - Reading Digital Data from Reed Click using Python Code

- Controlling the Actuator using BeagleBone Black Wireless (BBBW) Board

  - Understanding of Buzz 2 Click Hardware Connection

  - Controlling the Buzz 2 Click using Python Code

- Controlling the Actuator based on Digital Sensor Data using BeagleBone Black Wireless (BBBW) Board

  - Developing a Magnetic Door Alert System using Reed and Buzz 2 Click

  - Tinkering Time

# Reading the Digital Sensor Data using BeagleBone Black Wireless (BBBW) Board

## Understanding of Motion Click Hardware Connection

Motion Click is a motion detector sensitive only to live bodies. It carries PIR500B, a pyroelectric sensor which generates a voltage when exposed to infrared radiation emitted by live bodies. The white plastic Fresnel lens covering the sensor filters visible light. The signal is processed by a BISS0001 PIR sensor controller which sends an interrupt to the MCU. An onboard potentiometer lets you adjust the detecting range of the sensor (up to 1.7 meters).

Motion Click is ideal for alarm systems, light switch controllers, and similar systems where human presence needs to be detected.

Motion Click, and its respective schematic are shown in the Figure below.

Figure 1.1a: Motion Click and Schematic

- Connect the Motion Click to the mikroBUS cape and BBBW board as shown in the Figure below.

Figure 1.1b: Connecting Motion Click to mikroBUS Cape and BBBW Board

## Reading the Digital Data from Motion Click using Python Code

- Ensure that the BBBW board is powered up and connected to the computer through a USB cable. Open the web browser (preferably Chrome browser) and type “http://192.168.7.2:3000” in the address bar.

- Right click on the folder “MyFirstPythonProject” and select the “New File” from the drop-down menu to create a new python file. Name the file as “Motion.py”.

- Double click on the newly created file “Motion.py” and enter the following code into the file under the Editor section.

import time import Adafruit_BBIO.GPIO as GPIO GPIO.setup("P9_15", GPIO.IN) while True: if GPIO.input("P9_15"): print("Motion is Detected") else: print("No Motion is Detected") time.sleep(0.3)

- Click on the “Run” button located beside the Menu Tab to execute the “Motion.py” file. It is observed that text “No Motion is Detected” is printed at the output console window.

- Wave your hand on top of the Motion Click and it is observed that the red Led on the Motion Click is turned on and the text “Motion is Detected” is printed at the output console window.

- Stop waving your hand and it is observed the red led is turned off and the text “No Motion is Detected” is printed again at the output console window.

## Understanding of Vibra Sense Click Hardware Connection

Vibra Sense Click is a low-cost micro shock vibration sensor with a digital output which can be set as an Interrupt. An onboard potentiometer lets you set the interrupt threshold. It’s a very simple and effective solution for detecting vibrations. The sensing unit consists of a round housing in which a spring is coiled around a metal pin. When exposed to vibration, the spring contacts the pin and closes the switch.

Vibra Sense Click, and its respective schematic are shown in the Figure below.

Figure 1.3a: Vibra Sense Click and Schematic

- Connect the Vibra Sense Click to the mikroBUS cape and BBBW board as shown in the Figure below.

Figure 1.3b: Connecting Vibra Sense Click to mikroBUS Cape and BBBW board

## Reading the Digital Data from Vibra Sense Click using Python Code

- Ensure that the BBBW board is powered up and connected to the computer through a USB cable. Open the web browser (preferably Chrome browser) and type “http://192.168.7.2:3000” in the address bar.

- Right click on the folder “MyFirstPythonProject” and select the “New File” from the drop-down menu to create a new python file. Name the file as “VibraSense.py”.

- Double click on the newly created file “VibraSense.py” and enter the following code into the file under the Editor section.

import time import Adafruit_BBIO.GPIO as GPIO GPIO.setup("P9_23", GPIO.OUT) GPIO.setup("P9_41", GPIO.IN) GPIO.output("P9_23", GPIO.HIGH) while True: if GPIO.input("P9_41"): print("Vibration is Detected") else: print("No Vibration is Detected") time.sleep(0.3)

- Click on the “Run” button located beside the Menu Tab to execute the “VibraSense.py” file. It is observed that the text “No Vibration is Detected” is printed at the output at the output console window.

- Tune the Potentiometer on the Vibra Sense Click labelled as P1 towards clockwise to increase the sensitivity of the vibration sensor.

- Knock once on the table where the Vibra Sense Click is seated. It is observed that the red led is turned on and the text “Vibration is Detected” is printed at the output console window.

- It is also observed that the text “No Vibration is Detected” is printed again at the output console window and the red led is turned off once the knock is over.

## Understanding of IR Eclipse Click Hardware Connection

IR Eclipse Click carries an EE-SX198 photo interrupter sensor. This sensor consists of an infrared transmitter and receiver facing each other and spaced apart by a 3mm slit. When the beam from the transmitter is eclipsed by placing an object in the slit (like a piece of paper), the sensor is activated (indicated by the onboard INT LED). This type of sensor is typically used in printers, copiers, vending machines and so forth.

IR Eclipse Click and its respective schematic are shown in the Figure below.

Figure 1.5a: IR Eclipse Click and Schematic

- Connect the IR Eclipse Click to the mikroBUS cape and BBBW board as shown in the Figure below.

Figure 1.5b: Connecting IR Eclipse Click to mikroBUS Cape and BBBW Board

## Reading the Digital Data from IR Eclipse Click using Python Code

- Ensure that the BBBW board is powered up and connected to the computer through a USB cable. Open the web browser (preferably Chrome browser) and type “http://192.168.7.2:3000” in the address bar.

- Right click on the folder “MyFirstPythonProject” and select the “New File” from the drop-down menu to create a new python file. Name the file as “IREclipse.py”.

- Double click on the newly created file “IREclipse.py” and enter the following code into the file under the Editor section.

import time import Adafruit_BBIO.GPIO as GPIO GPIO.setup("P8_18", GPIO.IN) while True: if GPIO.input("P8_18"): print("Paper is Detected") else: print("No Paper is Detected") time.sleep(0.3)

- Click on the “Run” button located beside the Menu Tab to execute the “IREclipse.py” file. It is observed that the text “No Paper is Detected” is printed at the output console window.

- Put a piece of paper into the slit on the IR Eclipse Click. It is observed that the red led is turned on and the text “Paper is Detected” is printed at the output console window.

- Take away the paper and it is observed that the red led is turned off and the text “No Paper is Detected” is printed again at the output console window.

## Understanding of Reed Click Hardware Connection

Reed Click is a simple board that carries a standard (Single Pole Single Throw Normally Open) reed switch. A reed switch comprises of two thin magnetic contacts sealed inside a casing. One contact is a magnetic north pole, the other a south. The two contacts are separate, until a magnetic field is applied which snaps them close, activating the switch.

Reed Click, and its respective schematic are shown in the Figure below.

Figure 1.7a: Reed Click and Schematic

- Connect the Reed Click to the mikroBUS cape and BBBW board as shown in the Figure below.

Figure 1.7b: Connecting Reed Click to mikroBUS Cape and BBBW Board

## Reading the Digital Data from Reed Click using Python Code

- Ensure that the BBBW board is powered up and connected to the computer through a USB cable. Open the web browser (preferably Chrome browser) and type “http://192.168.7.2:3000” in the address bar.

- Right click on the folder “MyFirstPythonProject” and select the “New File” from the drop-down menu to create a new python file. Name the file as “Reed.py”.

- Double click on the newly created file “Reed.py” and enter the following code into the file under the Editor section.

import time import Adafruit_BBIO.GPIO as GPIO GPIO.setup("P8_10", GPIO.IN) while True: if GPIO.input("P8_10"): print("Magnet is Detected") else: print("No Magnet is Detected") time.sleep(0.3)

- Click on the “Run” button located beside the Menu Tab to execute the “Reed.py” file. It is observed that the text “No Magnet is Detected” is printed at the output console window.

- Bring a magnet given by the instructor closer to either the south or north pole of the reed switch located on the Reed Click. It is observed that the yellow led is turned on and the text “Magnet is Detected” is printed at the output console window.

- Bring away the magnet from the reed switch. It is observed that the yellow led is turned off and the text “No Magnet is Detected” is printed again at the output console window.

# Controlling the Actuator using BeagleBone Black Wireless (BBBW) Board

## Understanding of Buzz 2 Click Hardware Connection

Buzz 2 Click carries the CMT-8540S-SMT magnetic buzzer transducer. The buzzer’s resonant frequency is 4kHz. The CMT-8540S-SMT magnetic buzzer is controlled by the PWM signal where the signal frequency determines the sound pitch, and the duty cycle determines the sound volume.

Buzz 2 Click, and its respective schematic are shown in the Figure below.

Figure 2.1a: Buzz 2 Click and Schematic

- Connect the Buzz 2 Click to the mikroBUS cape and BBBW board as shown in Figure below.

Figure 2.1b: Connecting Buzz 2 Click to mikroBUS Cape and BBBW Board

## Controlling the Buzz 2 Click using Python Code

- Ensure that the BBBW board is powered up and connected to the computer through a USB cable. Open the web browser (preferably Chrome browser) and type “http://192.168.7.2:3000” in the address bar.

- Right click on the folder “MyFirstPythonProject” and select the “New File” from the drop-down menu to create a new python file. Name the file as “Buzz2.py”.

- Double click on the newly created file “Buzz2.py” and enter the following code into the file under the Editor section.

import time import Adafruit_BBIO.PWM as PWM PWM.start("P8_19", 50) PWM.set_frequency("P8_19", 523) time.sleep(0.5) PWM.set_frequency("P8_19", 587) time.sleep(0.5) PWM.set_frequency("P8_19", 659) time.sleep(0.5) PWM.stop("P8_19")

- Click on the “Run” button located beside the Menu Tab to execute the “Buzz2.py” file.

- Listen to the frequency tones of “Do Re Me” played by the Buzz 2 Click.

# Controlling the Actuator based on Sensor Data using BeagleBone Black Wireless (BBBW) Board

## Developing a Magnetic Door Alert System using Reed and Buzz 2 Click

- Connect both the Reed and Buzz 2 Clicks to the mikroBUS cape and BBBW board as shown in the Figure below.

Figure 3.1a: Connecting both the Reed and Buzz 2 Click to mikroBUS Cape and BBBW Board

- Ensure that the BBBW board is powered up and connected to the computer through a USB cable. Open the web browser (preferably Chrome browser) and type “http://192.168.7.2:3000” in the address bar.

- Right click on the folder “MyFirstPythonProject” and select the “New File” from the drop-down menu to create a new python file. Name the file as “MagDoorAlertSystem.py”.

- Double click on the newly created file “MagDoorAlertSystem.py” and start entering code into the file under the Editor section.

- Enter the code that imports all the necessary library to be used in the program as shown below.

import time import Adafruit_BBIO.GPIO as GPIO import Adafruit_BBIO.PWM as PWM

- Enter the code to initialize the required peripheral GPIO and PWM as shown below.

GPIO.setup("P8_10", GPIO.IN) PWM.start("P8_19", 50)

- Enter the main code that reads the sensor output from Reed Click and sound the Buzz 2 Click as shown below.

while True: if GPIO.input("P8_10"): PWM.stop("P8_19") else: PWM.start("P8_19", 50) PWM.set_frequency("P8_19", 1000) time.sleep(0.1) PWM.set_frequency("P8_19", 2000) time.sleep(0.1) time.sleep(0.3)

- Click on the “Run” button located beside the Menu Tab to execute the “MagDoorAlertSystem.py” file.

- The buzzer from the Buzz 2 Click will sound due to the absent of the magnet.

- Bring a magnet to closer to either the north of south pole of the reed switch on the Reed Click to stop the sound from buzzer.

- Compare the program output with your teammates and consult your lecturer for advice if it is not the same.

- Write the observation in the white box below for future reference if needed.

Right click and select “New comment” to insert your program as a comment.

## Tinkering Time

- Together with a classmate or two, think of a simple application that can use 1 of the 4 digital sensors clicks to capture an event and alert the user through Buzz 2 Click. Try to use other clicks you have learned before to complement your application.

- Connect the selected clicks to the mikroBUS cape and BBBW board.

- Create a python file in Cloud9 IDE and start writing your code.

- Present your complete work to your lecturer for advice.

- Share your work with your other classmates and teach them how you do it if they are interested.

Congratulations! You have successfully completed the Lab4a. Good job! Take a good break and stay tune for next lab. More exciting lab exercises coming to you!