Course			: Diploma in Electronics & Computer Engineering (EGDF20)

Module		: Connected System Design Project (EGE205)

Laboratory No.		: Lab 5b

Laboratory Title	: Networking: Setting up a Web Client using BeagleBone Black Wireless (BBBW) Board

Objective	: To use a BeagleBone Black Wireless (BBBW) Board to set up a web client.

Hardware Boards	: BBBW Board			x1

Pot Click			x1

Contents

- Setting Up a Web Client using BeagleBone Black Wireless (BBBW) Board

  - Understanding of Web Client and Socket.IO

  - Setting up a PC Web Server

  - Setting up the BBBW Board Web Client

  - Controlling and Monitoring the BBBW Board Web Client via a PC Web Server

  - Tinkering Time

# Setting Up a Web Client using BeagleBone Black Wireless (BBBW) Board

## Understanding of Web Client and Socket.IO Library

Web client is a piece of computer hardware or software that accesses a service made available by a server as part of the web client–server model of computer networks. For example, web browsers are web clients that connect to web servers and retrieve web pages for display. Email web clients retrieve email from mail servers. Multiplayer video games or online video games may run as a web client on each computer. The term "web client" may also be applied to computers or devices that run the web client software or users that use the web client software.

Figure 1.1a: Web client-server Model

Socket.IO is a JavaScript library for real-time web applications. It enables real-time, bi-directional communication between web clients and servers. Socket.IO primarily uses the WebSocket protocol with polling as a fallback option while providing the same interface. Although it can be used as simply a wrapper for WebSocket, it provides many more features, including broadcasting to multiple sockets, storing data associated with each client, and asynchronous I/O.

Flask-SocketIO gives Flask applications access to low latency bi-directional communications between the clients and the server. The client-side application can use any of the SocketIO client libraries in JavaScript, Python, C++, Java and Swift, or any other compatible client to establish a permanent connection to the server.

Figure 1.1b: Socket.IO Library

## Setting up a PC Web Server

Downloading and Installing the Python and Visual Studio Code (VS Code) Software

- Download the latest python software from www.python.org as shown in the Figure below.

Figure 1.2a: Downloading the Latest Python Software for Windows

- Double click on the installer, check on “Add Python 3.9 to Path” and click on “Install Now” to start the installation process. Follow the steps to complete the installation.

Figure 1.2b: Python Software Installation

- Download the Visual Studio Code (VS Code) software from https://code.visualstudio.com/ as shown in the Figure below.

Figure 1.2c: Downloading the Visual Studio Code (VS Code) Software for Windows

- Double click on the installer, select the radio button on “I accept the agreeement” and click on “Next” button as shown in the Figure below.

Figure 1.2d: VS Code Software Installation

- Click on “Next” button to use the default installation location as shown in the Figure below.

Figure 1.2e: VS Code Software Installation

- Click on “Next” button again to use the recommended name for the Start Menu Folder as shown in the Figure below.

Figure 1.2f: VS Code Software Installation

- Check on “Create a desktop icon” and “Add to PATH (requires shell restart). Click on “Next” button as shown in the Figure below.

Figure 1.2g: VS Code Software Installation

- Click on “Install” button to start the installation as shown in the Figure below.

Figure 1.2h: VS Code Software Installation

Setting Up the Web Server on the PC

- Create the following folders according to the folder structure below in the C: drive.

- MyFirstPythonProject

  - MyWebServer

    - templates

    - static

      - css

      - images

- In the PC, type in “cmd” in the window search function to search for the Command Prompt software and launch it as shown in the Figure below.

Figure 1.2i: Command Prompt

- Type in “pip install flask-socketio” and hit the “Enter” key to download and install the flask-socketio python library as shown in the Figure below.

Figure 1.2j: Command Prompt

- Type in “ipconfig” and hit the “Enter” key to retrieve the IP address of the wireless connection assigned to the PC. In this example, the IP address of 192.168.1.80 is assigned to the PC as shown in the Figure below.

Figure 1.2k: IP Address Assigned to the PC

- Type in “IDLE” in the window search function to search for the Python IDLE Shell software and launch it as shown in the Figure below.

Figure 1.2l: Python IDLE Shell

- Click on the “File” option from the menu toolbar located at the top of the IDLE Shell software and select “New File” from the drop-down menu option as shown in the Figure below.

Figure 1.2m: Create new python file

- Enter the following code into the newly created file. Modify the IP address of “192.168.X.X” according to the IP address you have retrieved from the PC earlier.

from flask import Flask from flask import render_template from flask_socketio import SocketIO from flask_socketio import emit app = Flask(__name__) socketio = SocketIO(app) @app.route('/') def index(): return render_template('index.html') @socketio.event def ControlUSR0Led(RxData): if RxData == 'on': socketio.emit('ControlUSR0Led', RxData) if RxData == 'off': socketio.emit('ControlUSR0Led', RxData) @socketio.event def BBBWEvent(RxData): socketio.emit('Web_BBBWEvent', RxData) print('Receive Data from BBBW') if __name__ == '__main__': app.run(host='192.168.X.X')

- Click on the “File” option from the menu toolbar located at the top of the IDLE Shell software and select “Save As…” from the drop-down menu option as shown in the Figure below. Navigate to the MyWebServer folder created earlier and save the file using the name of “WebServer.py”.

Figure 1.2n: Save the python file

- Click on the “Run” option from the menu toolbar located at the top of the IDLE Shell software and select “Run Module” from the drop-down menu option to execute the “WebServer.py” file as shown in the Figure below.

Figure 1.2o: Executing the python file

- A window pops up to indicate that the web server is up and running as shown in the Figure below.

Figure 1.2p: Web Server is Running

Setting Up the Web Pages

- Look for Visual Studio Code (VS Code) software by using the window search function and launch it as shown in the Figure below.

Figure 1.2q: Launching the Visual Studio Code (VS Code) Software

- Select File > Open Folder… from the top menu in the Visual Studio Code (VS Code) software as shown in the Figure below.

Figure 1.2r: Opening the Project Folder

- Navigate to the MyFirstPythonProject folder created earlier in the C: drive and click on the “Select Folder” button as shown in Figure below.

Figure 1.2s: Opening the Project Folder

- Ensure that the MyFirstPythonProject folder is opened and shown on the left panel of the Visual Studio Code (VS Code) software as shown in Figure below.

Figure 1.2t: Opening the Project Folder

- Expand the MyWebServer folder and right-click on the “css’’. Select “New File” from the drop-down menu to create a css file in the css folder and name it as “style.css” as shown in the Figure below.

Figure 1.2u: Create a css file

- Enter the following code into the “style.css” file and save the file.

body { font-family: Arial, Helvetica, sans-serif; font-size: 20px; color: gray; } button { border-radius: 30px; padding: 16px 32px; font-size: 16px; transition-duration: 0.4s; cursor: pointer; width: 100px; background-color: white; border: 2px solid #008CBA; } button:hover { background-color: #008CBA; color: white; } table, td { padding: 50px; text-align: center; border-collapse: collapse; border: 1px solid #ddd; } tr:nth-child(even) { background-color: #f2f2f2; } td:hover { background-color:#ddd; }

- Download the LightBulb.png file from Blackboard. Copy and paste the image to the images folder inside static folder created earlier.

- Create a html file in the templates folder and name it as “index.html”. Enter the following code into the file. Modify the IP address of “192.168.X.X” according to the IP address you have retrieved from the PC earlier.

<html> <head> <!Provide path to link to styling file, image files and 3rd party javascript library––> <link rel='stylesheet' href="{{ url_for('static', filename='css/style.css') }}"> <img id="lightbulb" src="{{url_for('static', filename='images/LightBulb.png')}}" style="display: none;"> <!Use of Javascript library socket.io.js to connect to the python web server––> <script src='https://ajax.googleapis.com/ajax/libs/jquery/3.5.1/jquery.min.js'></script> <script src='https://cdnjs.cloudflare.com/ajax/libs/socket.io/3.1.2/socket.io.js'></script> <script type='text/javascript'> $(document).ready(function(){ var socket = io.connect('http://192.168.X.X:5000'); var lightCanvas = document.getElementById("centerCanvas"); var lightContext = lightCanvas.getContext("2d"); var lightBulbImage = document.getElementById("lightbulb"); $('OnButton').click(function(){ socket.emit('ControlUSR0Led','on'); }); $('OffButton').click(function(){ socket.emit('ControlUSR0Led','off'); }); //Image Printing lightContext.drawImage(lightBulbImage, 175, 110); //Event triggered when python web server received data from BBBW1 socket.on('Web_BBBWEvent', function(RxData) { LightingControl_AdjustBrightness(RxData.data); console.log(RxData.data); }); //LightingControl_AdjustBrightness Function //Adjust the transparency level of the yellow circle based on the value received from the Pot Click function LightingControl_AdjustBrightness(BrightnessLevel) { lightContext.globalAlpha = 1.0; lightContext.clearRect(0, 0, 400, 300); lightContext.drawImage(lightBulbImage, 175, 110); lightContext.globalAlpha = BrightnessLevel; lightContext.arc(200, 150, 80, 0, 2 * Math.PI); lightContext.fillStyle = 'yellow'; lightContext.fill(); lightContext.globalAlpha = 1.0; lightContext.font = "15px Arial"; lightContext.fillStyle = 'Black'; lightContext.fillText("Estimated Lux Value:", 100, 260); lightContext.fillText(Math.floor(Math.abs(BrightnessLevel*400)), 260, 260); } }); </script> </head> <body> <center> <h1>Light Brightness Control System</h1> <table> <tr> <td> <h5>Controlling the BBBW Board</h5> <OnButton><button>ON</button></OnButton> <OffButton><button>OFF</button></OffButton> </td> </tr> <tr> <td> <canvas id="centerCanvas" width="400" height="300" style="border:1px solid #d3d3d3;" > Your browser does not support the HTML5 canvas tag. </canvas> </td> </tr> </table> <h5>Copyright @ EGE205 Connected System Project Design</h5> <center> </body> </html>

Accessing the Web Server

- Launch the internet browser of a PC (chrome) or mobile phone (safari) that are connected to the same Wi-Fi network in which the BBBW board is connected to.

- Type the following IP address with port number “192.168.X.X:5000” on the URL address bar of the browser and hit the Enter key. Replace the X in the IP address according to the IP address you have retrieved from the PC earlier. In this example, “192.168.86.64:5000” is used it is observed that the following user interface is printed on the browser as shown in Figure below.

Figure 1.2v: Accessing the Web Server on PC

## Setting up the BBBW Board Web Client

Setting up the BBBW Web Client

- Connect the Pot Click to the mikroBUS cape and BBBW board as shown in the Figure below.

Figure 1.3a: Connecting Pot Click to mikroBUS Cape and BBBW Board

Installation of Python Library

- Log in to the BBBW Board through SSH using the default username “debian” and password “temppwd”.

- Use the Connmanctl CLI command to connect the BBBW Board to the same Wi-Fi that the PC Web Server is connected to.

- Type in the command “pwd” and hit the “Enter” key. Ensure that the current working directory is “/home/debian” as shown in the Figure below.

Figure 1.3b: Current Working Directory

- Type in the command “ls” and hit the “Enter” key. Confirm that the folder named PythonLibrary has been created as shown in the Figure below. If it is not created, type in the command “mkdir PythonLibrary” to create the folder.

Figure 1.3c: Ensuring PythonLibrary Folder Is Created

- Type in the command “cd PythonLibrary” and hit the “Enter” key as shown in the Figure below.

Figure 1.3d: Accessing the PythonLibrary Folder

- Type in the command “git clone https://github.com/nypege205/Flask-SocketIO.git” and hit the “Enter” key. Type in the github username “nypege205” and password “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” and hit the “Enter” key again. A copy of the Flask-SocketIO Python Library is now cloned into the PythonLibrary folder as shown in the Figure below.

Figure 1.3e: Cloning the Flask-SocketIO Python Library

- Type in the command “ls” and hit the “Enter” key. A folder named Flask-SocketIO is listed as shown in the Figure below.

Figure 1.3f: Listing the Flask-SocketIO Python Library Folder

- Type in the command “cd Flask-SocketIO” and hit the “Enter” key as shown in the Figure below.

Figure 1.3g: Accessing the Flask-SocketIO Folder

- Type in the command “sudo python3 setup.py install” and hit the “Enter” key. Type in the password “temppwd” and hit the “Enter” key again. The installation may take up to 5 minutes. The completion of the installation process is shown in the Figure below.

Figure 1.3h: Completion of Flask-SocketIO Python Library Installation

- Type in the command “cd ..” and hit the “Enter” key to exit the Flask-SocketIO directory/folder as shown in the Figure below.

Figure 1.3i: Exit to Flask-SocketIO Folder

- Type in the command “git clone https://github.com/nypege205/requests.git” and hit the “Enter” key. Type in the github username “nypege205” and password “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” and hit the “Enter” key again. A copy of the Flask-SocketIO Python Library is now cloned into the PythonLibrary folder as shown in the Figure below.

Figure 1.3j: Cloning the requests Python Library

- Type in the command “ls” and hit the “Enter” key. A folder named “requests” is listed as shown in the Figure below.

Figure 1.3k: Listing the requests Python Library Folder

- Type in the command “cd requests” and hit the “Enter” key as shown in the Figure below.

Figure 1.3l: Accessing the Flask-SocketIO folder

- Type in the command “sudo python3 setup.py install” and hit the “Enter” key. Type in the password “temppwd” and hit the “Enter” key again. The installation may take up to 5 minutes. The completion of the installation process is shown in the Figure below.

Figure 1.3m: Completion of requests Python Library Installation

Creating and Executing Python File in Cloud9 IDE

- Open the web browser (preferably Chrome browser) and type “http://192.168.7.2:3000” in the address bar.

- Right click on the folder “MyFirstPythonProject” and select the “New File” from the drop-down menu to create a new html file. Name the file as “WebClient.py” as shown in the Figure below.

Figure 1.3o: Creating a New File

- Double click on the newly created python file “WebClient.py” and enter the following code into the file under the Editor section. Modify the IP address of “192.168.X.X” according to the IP address you have retrieved from the PC earlier.

import socketio import time import Adafruit_BBIO.GPIO as GPIO import Adafruit_BBIO.ADC as ADC GPIO.setup('USR0', GPIO.OUT) ADC.setup() sio = socketio.Client() @sio.event def connect(): print('Connection established.') @sio.event def disconnect(): print('Disconnected from server.') @sio.event def ControlUSR0Led(RxData): if RxData == 'on': GPIO.output('USR0', GPIO.HIGH) print('USD0 LED is ON!') if RxData == 'off': GPIO.output('USR0', GPIO.LOW) print('USD0 LED is OFF!') while True: try: sio.connect('http://192.168.X.X:5000') break except: print("Try to connect to the server.") pass OldDigitalValue = 0 while True: try: NewDigitalValue = ADC.read("P9_37") print("Digital Value: %f" % (NewDigitalValue)) if(abs(NewDigitalValue - OldDigitalValue) > 0.1): sio.emit('BBBWEvent', {'data': NewDigitalValue}) print('Data sent!') OldDigitalValue = NewDigitalValue except: print('Unable to transmit data.') pass time.sleep(0.5)

- Click on the “Run” button located beside the Menu Tab to execute the “WebClient.py” file.

## Controlling and Monitoring BBBW Board Web Client via a PC Web Server

- Ensure that the web server on PC is running.

- Ensure that the BBBW board is running.

- Turn the knob on the Pot Click and observe that the data (brightness color and lux value) from the BBBW board is streamed into the web server on PC as shown in the Figure below.

Figure 1.4a: Data Streamed into the Web Server on PC

- Click on the “ON” or “OFF” button on the webpage as shown in the Figure below. It is observed that that the USR0 led located at the BBBW board is turned on and off.

Figure 1.4b: Controlling the BBBW Board from the Web Server on PC

## Tinkering Time

- Together with a classmate or two, think of a simple application that can stream in data from the click board you have learnt and display it through the web server webpage.

- Connect the selected clicks to the mikroBUS cape and BBBW board.

- Create a python file in Cloud9 IDE and start writing your code.

- Present your complete work to your lecturer for advice.

- Share your work with your other classmates and teach them how you do it if they are interested.

Congratulations! You have successfully completed the Lab5b. Good job! You are now ready to develop a bigger scale connected system!