Course			: Diploma in Electronics & Computer Engineering (EGDF20)

Module		: Connected System Design Project (EGE205)

Laboratory No.		: Lab 6a

Laboratory Title	: Networking: Setting up a connected system using Multiple BeagleBone Black

Wireless (BBBW) Boards

Objective	: To use multiple BeagleBone Black Wireless (BBBW) Boards to set up a connected

system.

Hardware Boards	: BBBW Board			x4

Contents

- Setting Up a Connected System using Multiple BeagleBone Black Wireless (BBBW) Boards

  - Understanding of Connected Systems

  - Setting up a Python Web Server on a PC

  - Setting up Multiple Web Clients in Multiple BBBW Boards

  - Controlling and Monitoring Multiple BBBW Boards via Web Server on a PC

Note: This lab requires a team to work together to see the ultimate result! Team leader to complete section 1.2 while the rest of the team members to complete section 1.3. For section 1.3, each team member is required to choose only 1 of the BBBW code to be implemented in his / her BBBW board. Upon completion of section 1.2 and 1.3 respectively. The team can proceed to section 1.4 to see the ultimate result of a connected system. Good Luck!

# Setting Up a Connected System using Multiple BeagleBone Black Wireless (BBBW) Board

## Understanding of Connected System

Connected System can refer to a collection of interrelated systems consisting of IT devices, sensors, and actuators that can seamlessly interact with each other. By incorporating the functions of sensing, actuation, and control, these systems can measure, describe, and analyze data to make decisions and perform actions both with and without human intervention.

How does a typical connected system work?

A typical connected system ecosystem consists of a central web server and sensor devices that use embedded systems, such as processors, sensors, and communication hardware, to collect, send and act on data they acquire from their environments. They are all inter-connected and usually have a unique ID such as IP address.

Collecting the data is done by transmitting it from the sensor devices to a central web server where data are to be processed and analyzed. Sometimes, these sensor devices communicate with other related devices and act on the information they get from one another. The devices do most of the work without human intervention, although people can interact with the devices such as, to set them up, give them instructions or access the data.

The connectivity such as Wi-Fi or Bluetooth, networking, and communication protocols such as HTTP, TCP/IP used in a connected system largely depend on the specific applications deployed.

A typical setup of a connected system using BBBW boards is shown in the Figure below.

Figure 1.1a: Connected System setup using BBBW Boards

## Setting up a Python Web Server on a PC

Setting Up the Web Server on the PC

- Create the following folders according to the folder structure below in the C: drive.

- MyFirstPythonProject

  - MyWebServer

    - templates

    - static

      - css

- In the PC, type in “cmd” in the window search function to search for the Command Prompt software and launch it as shown in the Figure below.

Figure 1.2a: Command Prompt

- Skip step 4 if flask-socketio python library has been installed earlier.

- Type in “pip install flask-socketio” and hit the “Enter” key to download and install the flask-socketio python library as shown in the Figure below.

Figure 1.2b: Command Prompt

- Type in “ipconfig” and hit the “Enter” key to retrieve the IP address of the wireless connection assigned to the PC. In this example, the IP address of 192.168.1.80 is assigned to the PC as shown in the Figure below.

Figure 1.2c: IP Address Assigned to the PC

- Type in “IDLE” in the window search function to search for the Python IDLE Shell software and launch it as shown in the Figure below.

Figure 1.2d: Python IDLE Shell

- Click on the “File” option from the menu toolbar located at the top of the IDLE Shell software and select “New File” from the drop-down menu option as shown in the Figure below.

Figure 1.2e: Create new python file

- Enter the following code into the newly created file. Modify the IP address of “192.168.X.X” according to the IP address you have retrieved from the PC earlier.

from flask import Flask from flask import render_template from flask_socketio import SocketIO from flask_socketio import emit app = Flask(__name__) socketio = SocketIO(app) @app.route('/') def index(): return render_template('index.html') @socketio.event def ControlUSR0Led(RxData): if RxData == 'on': socketio.emit('ControlUSR0Led', RxData) if RxData == 'off': socketio.emit('ControlUSR0Led', RxData) @socketio.event def BBBW1Event(RxData): socketio.emit('Web_BBBW1Event', RxData) print('Receive Data from BBBW1') @socketio.event def BBBW2Event(RxData): socketio.emit('Web_BBBW2Event', RxData) print('Receive Data from BBBW2') @socketio.event def BBBW3Event(RxData): socketio.emit('Web_BBBW3Event', RxData) print('Receive Data from BBBW3') @socketio.event def BBBW4Event(RxData): socketio.emit('Web_BBBW4Event', RxData) print('Receive Data from BBBW4') if __name__ == '__main__': app.run(host='192.168.X.X')

- Click on the “File” option from the menu toolbar located at the top of the IDLE Shell software and select “Save As…” from the drop-down menu option as shown in the Figure below. Navigate to the MyWebServer folder created earlier and save the file using the name of “WebServer.py”.

Figure 1.2f: Save the python file

- Click on the “Run” option from the menu toolbar located at the top of the IDLE Shell software and select “Run Module” from the drop-down menu option to execute the “WebServer.py” file as shown in the Figure below.

Figure 1.2g: Executing the python file

- A window pops up to indicate that the web server is up and running as shown in the Figure below.

Figure 1.2h: Web Server is Running

Setting Up the Web Pages

- Look for Visual Studio Code (VS Code) software by using the window search function and launch it as shown in the Figure below.

Figure 1.2i: Launching the Visual Studio Code (VS Code) Software

- Select File > Open Folder… from the top menu in the Visual Studio Code (VS Code) software as shown in the Figure below.

Figure 1.2j: Opening the Project Folder

- Navigate to the MyFirstPythonProject folder created earlier in the C: drive and click on the “Select Folder” button as shown in Figure below.

Figure 1.2k: Opening the Project Folder

- Ensure that the MyFirstPythonProject folder is opened and shown on the left panel of the Visual Studio Code (VS Code) software as shown in Figure below.

Figure 1.2l: Opening the Project Folder

- Expand the MyWebServer folder and right-click on the “css’’. Select “New File” from the drop-down menu to create a css file in the css folder and name it as “style.css” as shown in the Figure below.

Figure 1.2m: Create a css file

- Enter the following code into the “style.css” file and save the file.

body { font-family: Arial, Helvetica, sans-serif; font-size: 20px; color: gray; } button { border-radius: 30px; padding: 16px 32px; font-size: 16px; transition-duration: 0.4s; cursor: pointer; width: 100px; background-color: white; border: 2px solid #008CBA; } button:hover { background-color: #008CBA; color: white; } table, td { padding: 50px; text-align: center; border-collapse: collapse; border: 1px solid #ddd; } tr:nth-child(even) { background-color: #f2f2f2; } td:hover { background-color:#ddd; }

- Create a html file in the templates folder and name it as “index.html”. Enter the following code into the file. Modify the IP address of “192.168.X.X” according to the IP address you have retrieved from the PC earlier.

<html> <head> <link rel='stylesheet' href="{{ url_for('static', filename='css/style.css') }}"> <script src='https://ajax.googleapis.com/ajax/libs/jquery/3.5.1/jquery.min.js'></script> <script src='https://cdnjs.cloudflare.com/ajax/libs/socket.io/3.1.2/socket.io.js'></script> <script type='text/javascript'> $(document).ready(function(){ var socket = io.connect('http://192.168.X.X:5000'); $('OnButton').click(function(){ socket.emit('ControlUSR0Led','on'); }); $('OffButton').click(function(){ socket.emit('ControlUSR0Led','off'); }); socket.on('Web_BBBW1Event', function(RxData) { $('bbbw1').text(RxData.data); }); socket.on('Web_BBBW2Event', function(RxData) { $('bbbw2').text(RxData.data); }); socket.on('Web_BBBW3Event', function(RxData) { $('bbbw3').text(RxData.data); }); socket.on('Web_BBBW4Event', function(RxData) { $('bbbw4').text(RxData.data); }); }); </script> </head> <body> <center> <h1>My First Connected System!</h1> <table style='width:100%'> <tr> <td colspan="2"> <h3>Controlling all the BBBW Boards</h3> <OnButton><button>ON</button></OnButton> <OffButton><button>OFF</button></OffButton> </td> </tr> <tr> <td style='width:50%'> <h3>BBBW 1</h3> <h4>I am a Random Number Generator!</h4> <p>Your Lucky Number is <bbbw1>???</bbbw1></p> </td> <td style='width:50%'> <h3>BBBW 2</h3> <h4>I am a Clock!</h4> <p>The Local Time Now is <bbbw2>???</bbbw2></p> </td> </tr> <tr> <td style='width:50%'> <h3>BBBW 3</h3> <h4>I am a Mathematician!</h4> <p><bbbw3>1 + 1 = 2</bbbw3></p> </td> <td style='width:50%'> <h3>BBBW 4</h3> <h4>I am a Weather Station!</h4> <p>The Current Temperature in Singapore is <bbbw4>???</bbbw4> C</p> </td> </tr> </table> <center> </body> </html>

Accessing the Web Server

- Launch the internet browser of a PC (chrome) or mobile phone (safari) that are connected to the same Wi-Fi network in which the BBBW board is connected to.

- Type the following IP address with port number “192.168.X.X:5000” on the URL address bar of the browser and hit the Enter key. Replace the X in the IP address according to the IP address you have retrieved from the PC earlier. In this example, “192.168.1.2:5000” is used it is observed that the following user interface is printed on the browser as shown in Figure below.

Figure 1.2n: Accessing the Web Server on PC

## Setting up Multiple Web Clients in Multiple BBBW Boards

Installation of Python Library

- Log in to the BBBW Board through SSH using the default username “debian” and password “temppwd”.

- Use the Connmanctl CLI command to connect the BBBW Board to the same Wi-Fi that the PC Web Server is connected to.

- Skip step 4 to 15 if Flask-SocketIO and requests python library has been downloaded and installed earlier.

- Type in the command “pwd” and hit the “Enter” key. Ensure that the current working directory is “/home/debian” as shown in the Figure below.

Figure 1.3a: Current Working Directory

- Type in the command “ls” and hit the “Enter” key. Confirm that the folder named PythonLibrary has been created as shown in the Figure below. If it is not created, type in the command “mkdir PythonLibrary” to create the folder.

Figure 1.3b: Ensuring PythonLibrary Folder Is Created

- Type in the command “cd PythonLibrary” and hit the “Enter” key as shown in the Figure below.

Figure 1.3c: Accessing the PythonLibrary Folder

- Type in the command “git clone https://github.com/nypege205/Flask-SocketIO.git” and hit the “Enter” key. Type in the github username “nypege205” and password “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” and hit the “Enter” key again. A copy of the Flask-SocketIO Python Library is now cloned into the PythonLibrary folder as shown in the Figure below.

Figure 1.3d: Cloning the Flask-SocketIO Python Library

- Type in the command “ls” and hit the “Enter” key. A folder named Flask-SocketIO is listed as shown in the Figure below.

Figure 1.3e: Listing the Flask-SocketIO Python Library Folder

- Type in the command “cd Flask-SocketIO” and hit the “Enter” key as shown in the Figure below.

Figure 1.3f: Accessing the Flask-SocketIO Folder

- Type in the command “sudo python3 setup.py install” and hit the “Enter” key. Type in the password “temppwd” and hit the “Enter” key again. The installation may take up to 5 minutes. The completion of the installation process is shown in the Figure below.

Figure 1.3g: Completion of Flask-SocketIO Python Library Installation

- Type in the command “cd ..” and hit the “Enter” key to exit the Flask-SocketIO directory/folder as shown in the Figure below.

Figure 1.3h: Exit to Flask-SocketIO Folder

- Type in the command “git clone https://github.com/nypege205/requests.git” and hit the “Enter” key. Type in the github username “nypege205” and password “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” and hit the “Enter” key again. A copy of the Flask-SocketIO Python Library is now cloned into the PythonLibrary folder as shown in the Figure below.

Figure 1.3i: Cloning the requests Python Library

- Type in the command “ls” and hit the “Enter” key. A folder named “requests” is listed as shown in the Figure below.

Figure 1.3j: Listing the requests Python Library Folder

- Type in the command “cd requests” and hit the “Enter” key as shown in the Figure below.

Figure 1.3k: Accessing the Flask-SocketIO folder

- Type in the command “sudo python3 setup.py install” and hit the “Enter” key. Type in the password “temppwd” and hit the “Enter” key again. The installation may take up to 5 minutes. The completion of the installation process is shown in the Figure below.

Figure 1.3l: Completion of requests Python Library Installation

Creating and Executing Python File in Cloud9 IDE

- Open the web browser (preferably Chrome browser) and type “http://192.168.7.2:3000” in the address bar.

- Right click on the folder “MyFirstPythonProject” and select the “New File” from the drop-down menu to create a new html file. Name the file as “WebClient.py” as shown in the Figure below.

Figure 1.3t: Creating a New File

- Double click on the newly created python file “WebClient.py” and enter the following code into the file under the Editor section. Modify the IP address of “192.168.X.X” according to the IP address you have retrieved from the PC earlier.

- Choose ONLY 1 of the following code to be implemented in the BBBW board. Discuss among the team members to avoid using the same code below.

For BBBW1 Board

import socketio import random import time import Adafruit_BBIO.GPIO as GPIO sio = socketio.Client() GPIO.setup('USR0', GPIO.OUT) @sio.event def connect(): print('Connection established.') @sio.event def disconnect(): print('Disconnected from server.') @sio.event def ControlUSR0Led(RxData): if RxData == 'on': GPIO.output('USR0', GPIO.HIGH) print('USD0 LED is ON!') if RxData == 'off': GPIO.output('USR0', GPIO.LOW) print('USD0 LED is OFF!') while True: try: sio.connect('http://192.168.X.X:5000') break except: print("Try to connect to the server.") pass while True: try: RandomNum = random.randint(0,999) sio.emit('BBBW1Event', {'data': RandomNum}) print('Data sent!') except: pass print('Unable to transmit data.') time.sleep(2)

For BBBW2 Board

import socketio import random import time import Adafruit_BBIO.GPIO as GPIO sio = socketio.Client() GPIO.setup('USR0', GPIO.OUT) @sio.event def connect(): print('Connection established.') @sio.event def disconnect(): print('Disconnected from server.') @sio.event def ControlUSR0Led(RxData): if RxData == 'on': GPIO.output('USR0', GPIO.HIGH) print('USD0 LED is ON!') if RxData == 'off': GPIO.output('USR0', GPIO.LOW) print('USD0 LED is OFF!') while True: try: sio.connect('http://192.168.X.X:5000') break except: print("Try to connect to the server.") pass while True: try: LocalTime = time.asctime(time.localtime(time.time())) sio.emit('BBBW2Event', {'data': LocalTime}) print('Data sent!') except: pass print('Unable to transmit data.') time.sleep(2)

For BBBW3 Board

import socketio import random import time import Adafruit_BBIO.GPIO as GPIO sio = socketio.Client() GPIO.setup('USR0', GPIO.OUT) @sio.event def connect(): print('Connection established.') @sio.event def disconnect(): print('Disconnected from server.') @sio.event def ControlUSR0Led(RxData): if RxData == 'on': GPIO.output('USR0', GPIO.HIGH) print('USD0 LED is ON!') if RxData == 'off': GPIO.output('USR0', GPIO.LOW) print('USD0 LED is OFF!') while True: try: sio.connect('http://192.168.X.X:5000') break except: print("Try to connect to the server.") pass while True: try: num1 = random.randint(0,999) num2 = random.randint(0,999) result = num1 + num2 TxData = '{:3d} + {:3d} = {:4d}' .format(num1, num2, result) sio.emit('BBBW3Event', {'data': TxData}) print('Data sent!') except: pass print('Unable to transmit data.') time.sleep(2)

For BBBW4 Board

import socketio import random import time import requests import json import Adafruit_BBIO.GPIO as GPIO sio = socketio.Client() GPIO.setup('USR0', GPIO.OUT) @sio.event def connect(): print('Connection established.') @sio.event def disconnect(): print('Disconnected from server.') @sio.event def ControlUSR0Led(RxData): if RxData == 'on': GPIO.output('USR0', GPIO.HIGH) print('USD0 LED is ON!') if RxData == 'off': GPIO.output('USR0', GPIO.LOW) print('USD0 LED is OFF!') while True: try: sio.connect('http://192.168.X.X:5000') break except: print("Try to connect to the server.") pass while True: try: TxData = requests.get("https://api.data.gov.sg/v1/environment/air-temperature") TxDataObject = json.loads(TxData.text) TemperatureReading = TxDataObject['items'] for TemperatureReadings in TemperatureReading: TemperatureValue = TemperatureReadings['readings'] sio.emit('BBBW4Event', {'data': TemperatureValue[0]["value"]}) print('Data sent!') except: pass print('Unable to transmit data.') time.sleep(2)

- Click on the “Run” button located beside the Menu Tab to execute the “WebClient.py” file.

- Ensure the web server on the PC is up and running. It is observed that the respective BBBW boards are sending data to the web server as shown in the Figure below.

Figure 1.3u: BBBW Sending Data to Web Server

Setting Python File to Run at Boot

- Type in the command “sudo crontab -e” and hit the enter key. Type in the password “temppwd” and hit the “Enter” key again. Cron is a task management tool built-in into Debian. Type “1” to choose the nano editor as shown in the Figure below.

Figure 1.3v: Accessing Crontab

- Add the text “@reboot python3 /var/lib/cloud9/MyFirstPythonProject/WebClient.py &” into the crontab using nano. This is to add the python file WebClient.py to be ran at boot as shown in the Figure below. Nano is a simple command-line text editor included in most Linux installations.

Figure 1.3w: Adding WebClient.py To Be Ran At Boot

- Press the “Ctrl + X” key followed by “y” and “Enter” key to save the modified file and exit the nano text-editor as shown in the Figure below.

Figure 1.3x: Exiting the nano Text-editor

- Type the command “sudo reboot” and hit the “Enter” key to reboot the BBBW board. The new settings will take effect on the next boot-up.

## Controlling and Monitoring Multiple BBBW Boards via Web Server on a PC

- Ensure that the web server on PC is running.

- Ensure that all the 4 BBBW boards are rebooted and is running.

- Observe that the data from the 4 BBBW boards are streamed into the web server on PC as shown in the Figure below.

Figure 1.4a: Data Streamed into the Web Server on PC

- Click on the “ON” or “OFF” button on the webpage as shown in the Figure below. It is observed that that the USR0 led located at all the 4 BBBW boards are turned on and off synchronously.

Figure 1.4b: Controlling the BBBW Boards from the Web Server on PC

Congratulations! You have successfully completed the Lab6a. Good job! You are now ready to develop your own connected system!