Course			: Diploma in Electronics & Computer Engineering (EGDF20)

Module		: Connected System Design Project (EGE205)

Laboratory No.		: Lab 6b

Laboratory Title	: A Practical Example of a Connected System – The Connected System Classroom

Objective	: To use multiple BeagleBone Black Wireless (BBBW) Boards to set up a connected

System in a classroom

Hardware Boards	: BBBW Board			x4

Motion Click			x1

Pot Click			x1

Analog Key Click		x1

Reed Click 			x1

Contents

- Setting Up a Connected System using Multiple BeagleBone Black Wireless (BBBW) Boards

  - Understanding of Connected Systems Classroom

  - Setting up a Python Web Server on a PC

  - Setting up Multiple Web Clients in Multiple BBBW Boards

  - Controlling and Monitoring Multiple BBBW Boards via Web Server on a PC

Note: This lab requires a team to work together to see how a connected system classroom works! Team leader to complete section 1.2 while the rest of the team members to complete section 1.3. For section 1.3, each team member is required to choose only 1 of the BBBW code to be implemented in his / her BBBW board. Upon completion of section 1.2 and 1.3 respectively. The team can proceed to section 1.4 to see the working connected system classroom. Good Luck!

# Setting Up a Connected System using Multiple BeagleBone Black Wireless (BBBW) Board

## Understanding of Connected System Classroom

Connected System Classroom consists of a central PC web server and 4 BBBW boards with each having a click board on it to collect data. They are all inter-connected through Wi-Fi to the access point and have been assigned a unique IP address.

The first BBBW board uses the Motion Click to detect the present of movement in the classroom. Upon detecting the present of movement, data will be sent to the PC web server dashboard to toggle a motion detection image. The second BBBW board uses the Pot Click to allow the lighting in the classroom to be adjusted. The brightness adjustment from the Pot Click is reflected real-time on the PC web server dashboard through a yellow filled circle with different transparency value.

The third BBBW board uses the Analog Key Click to capture the key pressed to unlock the door for accessing to the classroom. For every key pressed, the key number is sent to the PC web server to perform the matching analysis. Once the correct key is matched in this case the key “6”, the door will be unlocked automatically. This action is illustrated by a simple door unlocked animation shown in the PC web server.

The fourth BBBW board uses the Reed Click to perform door detection. The Reed Click constantly check on the present or absent of the magnet and relay the respective data to the PC web server only after the door is unlocked. When a magnet is brought closer to the Reed Click, it means the door is closed and the respective data is sent to the PC web server dashboard to display an image showing the door is close.

The setup of a connected system classroom using BBBW boards is shown in the Figure below.

Figure 1.1a: Connected System Classroom setup using BBBW Boards

## Setting up a Python Web Server on a PC

Setting Up the Web Server on the PC

- Create the following folders according to the folder structure below in the C: drive.

- ConnectedSystemClassroom

  - MyWebServer

    - templates

    - static

      - css

      - images

- In the PC, type in “cmd” in the window search function to search for the Command Prompt software and launch it as shown in the Figure below.

Figure 1.2a: Command Prompt

- Skip step 4 if flask-socketio python library has been installed earlier.

- Type in “pip install flask-socketio” and hit the “Enter” key to download and install the flask-socketio python library as shown in the Figure below.

Figure 1.2b: Command Prompt

- Type in “ipconfig” and hit the “Enter” key to retrieve the IP address of the wireless connection assigned to the PC. In this example, the IP address of 192.168.1.80 is assigned to the PC as shown in the Figure below.

Figure 1.2c: IP Address Assigned to the PC

- Type in “IDLE” in the window search function to search for the Python IDLE Shell software and launch it as shown in the Figure below.

Figure 1.2d: Python IDLE Shell

- Click on the “File” option from the menu toolbar located at the top of the IDLE Shell software and select “New File” from the drop-down menu option as shown in the Figure below.

Figure 1.2e: Create new python file

- Enter the following code into the newly created file. Modify the IP address of “192.168.X.X” according to the IP address you have retrieved from the PC earlier.

from flask import Flask from flask import render_template from flask_socketio import SocketIO from flask_socketio import emit app = Flask(__name__) socketio = SocketIO(app) @app.route('/') def index(): return render_template('index.html') @socketio.event def BBBW1Event(RxData): socketio.emit('Web_BBBW1Event', RxData) print('Receive Data from BBBW1') @socketio.event def BBBW2Event(RxData): socketio.emit('Web_BBBW2Event', RxData) print('Receive Data from BBBW2') @socketio.event def BBBW3Event(RxData): socketio.emit('Web_BBBW3Event', RxData) print('Receive Data from BBBW3') @socketio.event def BBBW4Event(RxData): socketio.emit('Web_BBBW4Event', RxData) print('Receive Data from BBBW4') if __name__ == '__main__': app.run(host='192.168.X.X')

- Click on the “File” option from the menu toolbar located at the top of the IDLE Shell software and select “Save As…” from the drop-down menu option as shown in the Figure below. Navigate to the MyWebServer folder created earlier and save the file using the name of “WebServer.py”.

Figure 1.2f: Save the python file

- Click on the “Run” option from the menu toolbar located at the top of the IDLE Shell software and select “Run Module” from the drop-down menu option to execute the “WebServer.py” file as shown in the Figure below.

Figure 1.2g: Executing the python file

- A window pops up to indicate that the web server is up and running as shown in the Figure below.

Figure 1.2h: Web Server is Running

Setting Up the Web Pages

- Look for Visual Studio Code (VS Code) software by using the window search function and launch it as shown in the Figure below.

Figure 1.2i: Launching the Visual Studio Code (VS Code) Software

- Select File > Open Folder… from the top menu in the Visual Studio Code (VS Code) software as shown in the Figure below.

Figure 1.2j: Opening the Project Folder

- Navigate to the ConnectedSystemClassroom folder created earlier in the C: drive and click on the “Select Folder” button as shown in Figure below.

Figure 1.2k: Opening the Project Folder

- Ensure that the ConnectedSystemClassroom folder is opened and shown on the left panel of the Visual Studio Code (VS Code) software as shown in Figure below.

Figure 1.2l: Opening the Project Folder

- Expand the MyWebServer folder and right-click on the “css’’. Select “New File” from the drop-down menu to create a css file in the css folder and name it as “style.css” as shown in the Figure below.

Figure 1.2m: Create a css file

- Enter the following code into the “style.css” file and save the file.

body { font-family: Arial, Helvetica, sans-serif; font-size: 20px; color: gray; } button { border-radius: 30px; padding: 16px 32px; font-size: 16px; transition-duration: 0.4s; cursor: pointer; width: 100px; background-color: white; border: 2px solid #008CBA; } button:hover { background-color: #008CBA; color: white; } table, td { padding: 50px; text-align: center; border-collapse: collapse; border: 1px solid #ddd; }

- Download the lab6b_images.zip file from Blackboard. Unzip the file and copy and paste all the 7 images to the images folder inside static folder created earlier.

- Create a html file in the templates folder and name it as “index.html”. Enter the following code into the file. Modify the IP address of “192.168.X.X” according to the IP address you have retrieved from the PC earlier.

<html> <head> <!Provide path to link to styling file, image files and 3rd party javascript library––> <link rel='stylesheet' href="{{ url_for('static', filename='css/style.css') }}"> <img id="floorplan" src="{{url_for('static', filename='images/Floorplan.png')}}" style="display: none;"> <img id="motionsensoriconblue" src="{{url_for('static', filename='images/MotionSensorIconBlue.png')}}" style="display: none;"> <img id="motionsensoriconred" src="{{url_for('static', filename='images/MotionSensorIconRed.png')}}" style="display: none;"> <img id="lightbulb" src="{{url_for('static', filename='images/LightBulb.png')}}" style="display: none;"> <img id="keypad" src="{{url_for('static', filename='images/Keypad.png')}}" style="display: none;"> <img id="dooropen" src="{{url_for('static', filename='images/DoorOpen.png')}}" style="display: none;"> <img id="doorclose" src="{{url_for('static', filename='images/DoorClose.png')}}" style="display: none;"> <!Use of Javascript library socket.io.js to connect to the python web server––> <script src='https://ajax.googleapis.com/ajax/libs/jquery/3.5.1/jquery.min.js'></script> <script src='https://cdnjs.cloudflare.com/ajax/libs/socket.io/3.1.2/socket.io.js'></script> <script type='text/javascript'> $(document).ready(function() { //Initialization and declaration of global variable var socket = io.connect('http://192.168.X.X:5000'); var floorPlanCanvas = document.getElementById("centerCanvas"); var floorPlanContext = floorPlanCanvas.getContext("2d"); var floorPlanImage = document.getElementById("floorplan"); var motionCanvas = document.getElementById("topLeftCanvas"); var motionContext = motionCanvas.getContext("2d"); var motionSensorIconBlueImage = document.getElementById("motionsensoriconblue"); var motionSensorIconRedImage = document.getElementById("motionsensoriconred"); var lightCanvas = document.getElementById("bottomLeftCanvas"); var lightContext = lightCanvas.getContext("2d"); var lightBulbImage = document.getElementById("lightbulb"); var doorAccessCanvas = document.getElementById("topRightCanvas"); var doorAccessContext = doorAccessCanvas.getContext("2d"); var keyPadImage = document.getElementById("keypad"); var doorDetectCanvas = document.getElementById("bottomRightCanvas"); var doorDetectContext = doorDetectCanvas.getContext("2d"); var doorOpenImage = document.getElementById("dooropen"); var doorCloseImage = document.getElementById("doorclose"); var BlinkingFlagStatus = 0; var DoorOpenTurning; var DoorRotateCount = 0; var DoorOpenCloseStatus = 0; //Image Printing floorPlanContext.drawImage(floorPlanImage, 0, 0); motionContext.drawImage(motionSensorIconBlueImage, 70, 37); lightContext.drawImage(lightBulbImage, 75, 50); doorAccessContext.drawImage(keyPadImage, 50, 0); doorDetectContext.drawImage(doorCloseImage, 50, 33); //Door Panel Printing floorPlanContext.beginPath(); floorPlanContext.lineWidth = 5; floorPlanContext.lineCap = "round"; floorPlanContext.moveTo(550,420); floorPlanContext.lineTo(550,375); floorPlanContext.stroke(); //Event triggered when python web server received data from BBBW1 socket.on('Web_BBBW1Event', function(RxData) { $('bbbw1').text(RxData.data); MotionDetection_BlinkImage(); }); //Event triggered when python web server received data from BBBW2 socket.on('Web_BBBW2Event', function(RxData) { $('bbbw2').text(RxData.data); LightingControl_AdjustBrightness(RxData.data); }); //Event triggered when python web server received data from BBBW3 socket.on('Web_BBBW3Event', function(RxData) { $('bbbw3').text(RxData.data); DoorAccess_KeyCheck(RxData.data); }); //Event triggered when python web server received data from BBBW4 socket.on('Web_BBBW4Event', function(RxData) { $('bbbw4').text(RxData.data); if(DoorOpenCloseStatus == 1) { DoorDetection_OpenCloseCheck(RxData.data); } }); //MotionDetection_BlinkImage Function //Toggle between motionSensorIconBlueImage & motionSensorIconRedImage when motion is detected function MotionDetection_BlinkImage() { //Clear the existing image motionContext.globalAlpha = 1.0; motionContext.clearRect(0, 0, 200, 155); if (BlinkingFlagStatus == 0) { BlinkingFlagStatus = 1; motionContext.drawImage(motionSensorIconRedImage, 70, 37); } else { BlinkingFlagStatus = 0; motionContext.drawImage(motionSensorIconBlueImage, 70, 37); } } //LightingControl_AdjustBrightness Function //Adjust the transparency level of the yellow circle based on the value received from the Pot Click function LightingControl_AdjustBrightness(BrightnessLevel) { lightContext.globalAlpha = 1.0; lightContext.clearRect(0, 0, 200, 155); lightContext.drawImage(lightBulbImage, 75, 50); lightContext.globalAlpha = BrightnessLevel; lightContext.arc(100, 70, 40, 0, 2 * Math.PI); lightContext.fillStyle = 'yellow'; lightContext.fill(); } //DoorAccess_KeyCheck Function //Highlight the individual number key based on the value received from the Analog Key Click function DoorAccess_KeyCheck(KeyPressed) { doorAccessContext.globalAlpha = 1.0; doorAccessContext.clearRect(0, 0, 200, 155); doorAccessContext.drawImage(keyPadImage, 50, 0); if(KeyPressed == 1) { doorAccessContext.beginPath(); doorAccessContext.globalAlpha = 0.5; doorAccessContext.arc(72, 20, 24, 0, 2 * Math.PI); doorAccessContext.fillStyle = 'white'; doorAccessContext.fill(); } if(KeyPressed == 2) { doorAccessContext.beginPath(); doorAccessContext.globalAlpha = 0.5; doorAccessContext.arc(127, 20, 24, 0, 2 * Math.PI); doorAccessContext.fillStyle = 'white'; doorAccessContext.fill(); } if(KeyPressed == 3) { doorAccessContext.beginPath(); doorAccessContext.globalAlpha = 0.5; doorAccessContext.arc(72, 75, 24, 0, 2 * Math.PI); doorAccessContext.fillStyle = 'white'; doorAccessContext.fill(); } if(KeyPressed == 4) { doorAccessContext.beginPath(); doorAccessContext.globalAlpha = 0.5; doorAccessContext.arc(127, 75, 24, 0, 2 * Math.PI); doorAccessContext.fillStyle = 'white'; doorAccessContext.fill(); } if(KeyPressed == 5) { doorAccessContext.beginPath(); doorAccessContext.globalAlpha = 0.5; doorAccessContext.arc(72, 130, 24, 0, 2 * Math.PI); doorAccessContext.fillStyle = 'white'; doorAccessContext.fill(); } if(KeyPressed == 6) { doorAccessContext.beginPath(); doorAccessContext.globalAlpha = 0.5; doorAccessContext.arc(127, 130, 24, 0, 2 * Math.PI); doorAccessContext.fillStyle = 'white'; doorAccessContext.fill(); floorPlanContext.translate(550, 420); DoorOpenTurning = setInterval(DoorOpenTurningAnimation, 50); } } //DoorOpenTurningAnimation Function function DoorOpenTurningAnimation() { floorPlanContext.globalAlpha = 1.0; floorPlanContext.clearRect(-10, 0, 60, -53); floorPlanContext.beginPath(); floorPlanContext.lineWidth = 5; floorPlanContext.lineCap = "round"; floorPlanContext.moveTo(0,0); floorPlanContext.lineTo(0,-45); floorPlanContext.stroke(); floorPlanContext.rotate(0.1); DoorRotateCount++; if (DoorRotateCount == 17) { DoorRotateCount = 0; floorPlanContext.rotate(-1.7); floorPlanContext.translate(-550, -420); floorPlanContext.globalAlpha = 1.0; doorDetectContext.clearRect(0, 0, 200, 155); doorDetectContext.drawImage(doorOpenImage, 50, 28); DoorOpenCloseStatus = 1; clearInterval(DoorOpenTurning); } } //DoorDetection_OpenCloseCheck Function function DoorDetection_OpenCloseCheck(DoorDetectionStatus) { if (DoorDetectionStatus == 1) { doorDetectContext.globalAlpha = 1.0; doorDetectContext.clearRect(0, 0, 200, 155); doorDetectContext.drawImage(doorCloseImage, 50, 33); } else { doorDetectContext.globalAlpha = 1.0; doorDetectContext.clearRect(0, 0, 200, 155); doorDetectContext.drawImage(doorOpenImage, 50, 28); } } }); </script> </head> <body> <center> <h1>Connected System Classroom</h1> <table> <tr> <td> <h3>Motion Detection</h3> <canvas id="topLeftCanvas" width="200" height="155"> Your browser does not support the HTML5 canvas tag. </canvas> </td> <td rowspan = "2"> <canvas id="centerCanvas" width="620" height="440" style="border:1px solid #d3d3d3;" > Your browser does not support the HTML5 canvas tag. </canvas> </td> <td> <h3>Door Access</h3> <canvas id="topRightCanvas" width="200" height="155"> Your browser does not support the HTML5 canvas tag. </canvas> </td> </tr> <tr> <td> <h3>Lighting Control</h3> <canvas id="bottomLeftCanvas" width="200" height="155"> Your browser does not support the HTML5 canvas tag. </canvas> </td> <td> <h3>Door Detection</h3> <canvas id="bottomRightCanvas" width="200" height="155" > Your browser does not support the HTML5 canvas tag. </canvas> </td> </tr> </table> <h5>Copyright @ EGE205 Connected System Project Design</h5> <center> </body> </html>

Accessing the Web Server

- Launch the internet browser of a PC (chrome) or mobile phone (safari) that are connected to the same Wi-Fi network in which the BBBW board is connected to.

- Type the following IP address with port number “192.168.X.X:5000” on the URL address bar of the browser and hit the Enter key. Replace the X in the IP address according to the IP address you have retrieved from the PC earlier. In this example, “192.168.1.80:5000” is used it is observed that the following user interface is printed on the browser as shown in Figure below.

Figure 1.2n: Accessing the Web Server on PC

## Setting up Multiple Web Clients in Multiple BBBW Boards

Setting up the Individual BBBW Boards

- Choose ONLY 1 of the BBBW boards setup as shown below to be implemented in your BBBW board. Discuss among the team members to avoid using the same setup below.

For BBBW1 Board

Figure 1.3a: Connecting Motion Click to mikroBUS Cape and BBBW Board

For BBBW2 Board

Figure 1.3b: Connecting Pot Click to mikroBUS Cape and BBBW Board

For BBBW3 Board

Figure 1.3c: Connecting Analog Key Click to mikroBUS Cape and BBBW Board

For BBBW4 Board

Figure 1.3d: Connecting Reed Click to mikroBUS Cape and BBBW Board

Installation of Python Library

- Log in to the BBBW Board through SSH using the default username “debian” and password “temppwd”.

- Use the Connmanctl CLI command to connect the BBBW Board to the same Wi-Fi that the PC Web Server is connected to.

- Skip step 4 to 15 if Flask-SocketIO and requests python library has been downloaded and installed earlier.

- Type in the command “pwd” and hit the “Enter” key. Ensure that the current working directory is “/home/debian” as shown in the Figure below.

Figure 1.3e: Current Working Directory

- Type in the command “ls” and hit the “Enter” key. Confirm that the folder named PythonLibrary has been created as shown in the Figure below. If it is not created, type in the command “mkdir PythonLibrary” to create the folder.

Figure 1.3f: Ensuring PythonLibrary Folder Is Created

- Type in the command “cd PythonLibrary” and hit the “Enter” key as shown in the Figure below.

Figure 1.3g: Accessing the PythonLibrary Folder

- Type in the command “git clone https://github.com/nypege205/Flask-SocketIO.git” and hit the “Enter” key. Type in the github username “nypege205” and password “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” and hit the “Enter” key again. A copy of the Flask-SocketIO Python Library is now cloned into the PythonLibrary folder as shown in the Figure below.

Figure 1.3h: Cloning the Flask-SocketIO Python Library

- Type in the command “ls” and hit the “Enter” key. A folder named Flask-SocketIO is listed as shown in the Figure below.

Figure 1.3i: Listing the Flask-SocketIO Python Library Folder

- Type in the command “cd Flask-SocketIO” and hit the “Enter” key as shown in the Figure below.

Figure 1.3j: Accessing the Flask-SocketIO Folder

- Type in the command “sudo python3 setup.py install” and hit the “Enter” key. Type in the password “temppwd” and hit the “Enter” key again. The installation may take up to 5 minutes. The completion of the installation process is shown in the Figure below.

Figure 1.3k: Completion of Flask-SocketIO Python Library Installation

- Type in the command “cd ..” and hit the “Enter” key to exit the Flask-SocketIO directory/folder as shown in the Figure below.

Figure 1.3l: Exit to Flask-SocketIO Folder

- Type in the command “git clone https://github.com/nypege205/requests.git” and hit the “Enter” key. Type in the github username “nypege205” and password “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” and hit the “Enter” key again. A copy of the Flask-SocketIO Python Library is now cloned into the PythonLibrary folder as shown in the Figure below.

Figure 1.3m: Cloning the requests Python Library

- Type in the command “ls” and hit the “Enter” key. A folder named “requests” is listed as shown in the Figure below.

Figure 1.3n: Listing the requests Python Library Folder

- Type in the command “cd requests” and hit the “Enter” key as shown in the Figure below.

Figure 1.3o: Accessing the Flask-SocketIO folder

- Type in the command “sudo python3 setup.py install” and hit the “Enter” key. Type in the password “temppwd” and hit the “Enter” key again. The installation may take up to 5 minutes. The completion of the installation process is shown in the Figure below.

Figure 1.3p: Completion of requests Python Library Installation

Creating and Executing Python File in Cloud9 IDE

- Open the web browser (preferably Chrome browser) and type “http://192.168.7.2:3000” in the address bar.

- Right click on the folder “MyFirstPythonProject” and select the “New File” from the drop-down menu to create a new html file. Name the file as “WebClient.py” as shown in the Figure below.

Figure 1.3q: Creating a New File

- Double click on the newly created python file “WebClient.py” and enter the following code into the file under the Editor section. Modify the IP address of “192.168.X.X” according to the IP address you have retrieved from the PC earlier.

- Choose ONLY 1 of the following code according to the board setup you have chosen earlier to be implemented in your BBBW board.

For BBBW1 Board

import socketio import time import Adafruit_BBIO.GPIO as GPIO sio = socketio.Client() GPIO.setup("P9_15", GPIO.IN) @sio.event def connect(): print('Connection established.') @sio.event def disconnect(): print('Disconnected from server.') while True: try: sio.connect('http://192.168.X.X:5000') break except: print("Try to connect to the server.") pass MotionDetectionStatus = 0 while True: try: MotionDetectionStatus = GPIO.input("P9_15") if MotionDetectionStatus: sio.emit('BBBW1Event', {'data': MotionDetectionStatus}) print('Data sent!') print("Motion is Detected") else: print("No Motion is Detected") except: print('Unable to transmit data.') pass time.sleep(0.5)

For BBBW2 Board

import socketio import time import Adafruit_BBIO.ADC as ADC sio = socketio.Client() ADC.setup() @sio.event def connect(): print('Connection established.') @sio.event def disconnect(): print('Disconnected from server.') while True: try: sio.connect('http://192.168.X.X:5000') break except: print("Try to connect to the server.") pass OldDigitalValue = 0 while True: try: NewDigitalValue = ADC.read("P9_37") print("Digital Value: %f" % (NewDigitalValue)) if(abs(NewDigitalValue - OldDigitalValue) > 0.1): sio.emit('BBBW2Event', {'data': NewDigitalValue}) print('Data sent!') OldDigitalValue = NewDigitalValue except: print('Unable to transmit data.') pass time.sleep(0.5)

For BBBW3 Board

import socketio import time import Adafruit_BBIO.ADC as ADC sio = socketio.Client() ADC.setup() @sio.event def connect(): print('Connection established.') @sio.event def disconnect(): print('Disconnected from server.') while True: try: sio.connect('http://192.168.X.X:5000') break except: print("Try to connect to the server.") pass KeyPressed = 0 while True: try: DigitalValue = ADC.read("P9_40") if DigitalValue < 0.100: print("No Key is Pressed") KeyPressed = 0 elif DigitalValue < 0.175: print("T6 Key is Pressed") KeyPressed = 6 elif DigitalValue < 0.350: print("T5 Key is Pressed") KeyPressed = 5 elif DigitalValue < 0.525: print("T4 Key is Pressed") KeyPressed = 4 elif DigitalValue < 0.700: print("T3 Key is Pressed") KeyPressed = 3 elif DigitalValue < 0.875: print("T2 Key is Pressed") KeyPressed = 2 elif DigitalValue == 1.00: print("T1 Key is Pressed") KeyPressed = 1 if(KeyPressed != 0): sio.emit('BBBW3Event', {'data': KeyPressed}) print('Data sent!') except: print('Unable to transmit data.') pass time.sleep(1)

For BBBW4 Board

import socketio import time import Adafruit_BBIO.GPIO as GPIO sio = socketio.Client() GPIO.setup("P8_10", GPIO.IN) @sio.event def connect(): print('Connection established.') @sio.event def disconnect(): print('Disconnected from server.') while True: try: sio.connect('http://192.168.X.X:5000') break except: print("Try to connect to the server.") pass PreviousDoorDetectionStatus = 0 while True: try: CurrentDoorDetectionStatus = GPIO.input("P8_10") if CurrentDoorDetectionStatus: print("Magnet is Detected (Door Closed)") else: print("No Magnet is Detected (Door Opened)") if (abs(CurrentDoorDetectionStatus - PreviousDoorDetectionStatus) > 0): sio.emit('BBBW4Event', {'data': CurrentDoorDetectionStatus}) print('Data sent!') PreviousDoorDetectionStatus = CurrentDoorDetectionStatus except: print('Unable to transmit data.') pass time.sleep(0.5)

- Click on the “Run” button located beside the Menu Tab to execute the “WebClient.py” file.

- Ensure the web server on the PC is up and running as shown in the Figure below.

Figure 1.3r: PC Web Server

- Wave your hand in front of the Motion Click or tune the potentiometer on the Pot Click or press the key “6” on the Analog Key Click then bring a magnet closer to the Reed Click. It is observed that the respective BBBW boards are sending data to the web server dashboard.

Setting Python File to Run at Boot

- Type in the command “sudo crontab -e” and hit the enter key. Type in the password “temppwd” and hit the “Enter” key again. Cron is a task management tool built-in into Debian. Type “1” to choose the nano editor as shown in the Figure below.

Figure 1.3s: Accessing Crontab

- Add the text “@reboot python3 /var/lib/cloud9/MyFirstPythonProject/WebClient.py &” into the crontab using nano. This is to add the python file WebClient.py to be ran at boot as shown in the Figure below. Nano is a simple command-line text editor included in most Linux installations.

Figure 1.3t: Adding WebClient.py To Be Ran At Boot

- Press the “Ctrl + X” key followed by “y” and “Enter” key to save the modified file and exit the nano text-editor as shown in the Figure below.

Figure 1.3u: Exiting the nano Text-editor

- Type the command “sudo reboot” and hit the “Enter” key to reboot the BBBW board. The new settings will take effect on the next boot-up.

## Controlling and Monitoring Multiple BBBW Boards via Web Server on a PC

- Ensure that the web server on PC is running.

- Ensure that all the 4 BBBW boards are rebooted and is running.

- Wave your hand in front of the Motion Click, tune the potentiometer on the Pot Click, press the key “6” on the Analog Key Click and bring a magnet closer to the Reed Click. It is observed that the respective BBBW boards are sending data to the web server dashboard as shown in the Figure below.

Figure 1.4a: Data Streamed into the Web Server on PC

Congratulations! You have successfully completed the Lab6b. Good job! You should be now having higher confidence to develop your own connected system!